# PA-28-161 CHECKLIST

## PREFLIGHT INSPECTION

#### Aircraft documents	VALID AND COMPLETE.

#### Control wheel	RELEASE BELTS.

#### Parking brake	ON.

#### Master switch	ON.

#### Magnetos	OFF.

#### Fuel	SELECT EMPTIER TANK.

#### Fuel quantity gauges	NOTE READINGS.

#### Lights	ON.

#### Stall warning vane	OPERATE.

#### Overhead switches	OFF*.

#### Master switch	OFF*.

#### Fire extinguisher	IN PLACE.

#### First aid kit	IN PLACE.

#### Flaps	DOWN.		* Except night flying.

## WALK AROUND CHECKS – STARBOARD WING.

#### Flap			CHECK SECURITY AND ATTACHMENT.

#### Upper wing surface	CHECK CONDITION AND FREE FROM ICE/SNOW/FROST.

#### Aileron		CHECK CONDITION AND FREEDOM OF MOVEMENT.

#### Nav. and strobe lights	SECURE

#### Leading edge		CHECK CONDITION.

#### Lower wing surface	CHECK CONDITION AND CLEANLINESS.

#### Fuel tank sump		SAMPLE/DRAIN FUEL STRAINER.

#### Fuel tank		DIP CONTENTS, SECURE CAP.

#### Fuel vent		OPEN.

#### Tyre			CHECK CONDITION, INFLATION AND CREEP.

#### Brake blocks		CHECK.

#### Oleo			CHECK EXTENSION 4.5 INCHES.

## NOSE GEAR

#### Tyre			CHECK CONDITION, INFLATION AND CREEP.

#### Oleo			CHECK EXTENSION 3.25 INCHES.

#### Torque link		SECURITY OF LINKAGES.

## FRONT FUSELAGE AND ENGINE

#### Windscreen		CHECK CONDITION AND CLEANLINESS.

#### Engine		OIL LEVEL BETWEEN 6 AND 8 QUARTS, CHECK FILLER CAP FINGER TIGHT

#### Engine gaskets, leads, primer lines CHECK FOR LEAKS AND SECURITY.

#### Engine panel		SECURE.

#### Cooling and air intakes	CLEAR.

#### Alternator belt		CHECK TENSION.

#### Propeller blades and spinner CHECK FOR NICKS, DAMAGE AND SECURITY.

#### Hydraulic fluid reservoir	CHECK LEVEL (FIRST FLIGHT OF THE DAY).

#### Engine gaskets, leads, primer lines CHECK FOR LEAKS AND SECURITY.

#### Engine panel		SECURE.

#### Fuel strainer		SAMPLE/DRAIN.

## PORT WING.

#### Fuel tank		DIP CONTENTS, SECURE CAP

#### Fuel vent		OPEN.

#### Upper wing surface	CHECK CONDITION, FREE FROM ICE/SNOW/FROST.

#### Leading edge		CHECK CONDITION.

#### Lower wing surface	CHECK CONDITION AND CLEANLINESS.

#### Fuel tank sump		SAMPLE/DRAIN FUEL STRAINER AFTER REFUELLING.

#### Tyre			CHECK CONDITION, INFLATION AND CREEP.

#### Oleo			CHECK EXTENSION 4.5 INCHES.

#### Nav. and strobe lights	SECURE

#### Aileron		CHECK CONDITION, SECURITY AND FREEDOM OF MOVEMENT.

#### Flap			CHECK SECURITY AND ATTACHMENT.

## PORT FUSELAGE

#### Fuselage skin		CHECK CONDITION.

#### Static vent		CLEAR.

#### Radio aerials		SECURE.

#### Fin			SECURE.

#### Stabilator and trim tab	CHECK CONDITION AND FREEDOM OF MOVEMENT.

#### Rudder		CHECK FIXINGS, CABLE AND HINGES SECURE.

#### Lights		CHECK SECURITY.

## STARBOARD FUSELAGE

#### Static vent		CLEAR.

#### Fuselage skin		CONDITION.

## INTERNAL.

#### Top latch		OPEN.

#### Door			CLOSED, TOP LATCH CLOSED.

#### Seat			ADJUSTED, HARNESSES SECURE.

#### Parking brake		ON.

#### Trimmer		SLIGHTLY AFT OF NEUTRAL.

#### Flap			SELECT HALF THEN UP.

#### Flying controls		FULL AND FREE MOVEMENT IN THE CORRECT SENSE.

#### Alternate static		NORMAL.

#### Master/Alternator switches	ON, ALTERNATOR WARNING LIGHT ON.

#### Carb. Heat		COLD.

#### Fuel			SELECT EMPTIER TANK, CHECK CONTENTS.

#### Fuel pump		ON, CHECK PRESSURE READING THEN OFF.

#### Throttle		CHECK MOVEMENT, CLOSE.

#### Mixture		CHECK MOVEMENT, SET RICH.

#### Fin light		ON.

#### Landing light		OFF.

#### Strobes		OFF.

#### Pitot heat		OFF.

#### Radio and Navaids	OFF.

#### Nav. and panel lights	OFF.

#### Radio lights		OFF.

#### Circuit breakers		ALL IN.

#### Instruments and gauges	CLOCK ACCURATE, ASI VERTICAL, ALTIMETER QFE READING WITHIN +50' and -75'

#### Turn coordinator	FUNCTIONING.

#### VSI			Zero  200’ (NOTE ERROR).

#### Compass		SENSIBLE HEADING, NO DISCOLORATION AND NO BUBBLES.

#### OAT			SENSIBLE READING.

#### Hobbs meter		NOTE READING.

## PASSENGER BRIEF

#### The fire extinguisher is here and the medical kit here. This is where your seat harness buckle is. This is where the door lock is. Here is how you open it. To get out in a hurry exit to the rear and meet me 50 yards in front of the aircraft. In a crash take up the brace position with arms across your face.

## STARTING

#### Primer		MINIMUM THREE SHOTS WHEN REQUIRED, THEN LOCK.

#### Throttle		SET ¼” OPEN.

#### Area			LOOKOUT, SHOUT ‘CLEAR PROP’.

#### Starter		OPERATE (RELEASE AS SOON AS ENGINE FIRES).

#### Starter warning light	OUT

#### T’s and P’s		RISING

#### Low voltage warning light	OUT.

#### Power		SET 1200 RPM.

#### Flaps		UP

## AFTER STARTING CHECKS.

#### Oil pressure		RISING WITHIN 30 SECONDS.

#### Fuel pressure		IN THE GREEN AND ABOVE PREVIOUS READING.

#### Ammeter		NORMAL READING, MAX. 30 AMPS.

#### Suction		INDICATING.

#### Magnetos		CHECK FOR DEAD CUT.

#### Radio		INTERCOM AND PHONES ON.

#### Navaids		CHECK, IDENT AND SET.

#### Pitot Heat		ON.

#### DI			SYNCHRONISE WITH COMPASS.

#### AI			SET.

#### Pitot Heat		OFF.

#### Landing light		ON.

#### Clearance		REQUEST RADIO CHECK AND TAXI VFR FOR ……….

## TAXIING CHECKS.

#### Brakes		CHECK OPERATION GENTLY (OFFER CONTROL TO FE).

#### Rudder and nose gear steering FULL AND FREE MOVEMENT.

#### Flight instruments	WINGS ROLL RIGHT, BALL OUT TO THE LEFT, DI INCREASES, COMPASS ALSO.  WINGS ROLL LEFT, BALL OUT TO THE RIGHT, DI DECREASES, COMPASS ALSO. AI STEADY, ALTIMETER STEADY.

## POWER CHECKS.

#### Position		INTO WIND AND NOSEWHEEL STRAIGHT.

#### Fuel			CHANGE TO FULLER TANK.

#### Throttle		SET 2000 RPM, CHECK BRAKES HOLDING.

#### Carb. Heat		OPERATE 10 SECONDS, CHECK MAX. DROP 75 RPM.

#### Magnetos		CHECK LEFT AND RIGHT DROP DOES NOT EXCEED 175 RPM. MAX. DIFFERENCE 50 RPM.

#### Oil T’s & P’s		STEADY.

#### Annunciator panel	PRESS TO TEST.

#### Alternator load		AMMETER READING NORMAL (MAX. 30 AMPS.)

#### Suction		5” 0.1”

#### Throttle		CLOSE, CHECK IDLE BETWEEN 500 AND 700 RPM, RESET 1200 RPM. DO NOT IDLE BELOW 1200 RPM.

## ALTIMETER SETTINGS

#### Circuits		ET QFE, CHECK ±50'.

#### Local		SET AERODROME QNH, CHECK ±50'..

#### Cruise		SET ZONE/TMA/REGIONAL QNH BELOW 3000’. ABOVE TRANSITION LEVEL SET QNE.  FLY QUADRANTAL RULES.

#### Circuit join		SET QFE WITH AIRFIELD IN SIGHT.

#### Missed Approach	SET AERODROME QNH.

## PRE TAKE OFF CHECKS.

#### Trimmer		SET JUST AFT OF NEUTRAL.

#### Throttle		FRICTION NUT FINGER TIGHT.

#### Mixture		RICH, CARB. HEAT COLD.

#### Magnetos		BOTH.

#### Fuel			ON FULLEST TANK AND SUFFICIENT, FUEL PUMP ON, PRIMER IN AND LOCKED.

#### Flaps		25/SECOND NOTCH.

#### Gyros		AI ERECT, PITOT HEAT ON, DI SYNCHRONISED.

#### Hatches		CLOSED AND LATCHED.

#### Harnesses		SECURE.

#### Controls		FULL AND FREE MOVEMENT

## CAPTAIN’S TAKE OFF BRIEF.

#### This will be a normal take off with a noise abatement turn to the right. If there is an engine failure before Vr, I will close the throttle, apply brakes and vacate the runway if possible. If there is an engine failure after take off I will asses the situation and make a decision asking you for assistance if necessary.

## DURING TAKE OFF CHECKS.

#### DI			ALIGNED WITH RUNWAY.

#### Heels		WELL BACK TO AVOID TOE BRAKES.

#### Power		FULL THROTTLE, RPM 2400, T’S & P’S GREEN.

#### ASI			INCREASING.

## AFTER TAKE OFF CHECKS.

#### Brakes		APPLY MOMENTARILY.

#### Lookout		SELECT POINT LEFT OF NOSE.

#### Flaps		SLOWLY RETRACT AT 200’ (ACCELERATE TO 79 KIAS).

#### Fuel pump		OFF ABOVE 1200’, CHECK PRESSURE MAINTAINED.

#### Landing light		OFF.

#### Radio		SELECT FREQUENCY REQUIRED.

#### Altimeter		SET AS REQUIRED.

## TOP OF CLIMB.

#### Fuel			CHECK CONTENTS, SELECT APPROPRIATE TANK.

#### Engine		T’s & P’S IN THE GREEN, CARB. HEAT FOR 10 SECONDS.

#### Mixture		LEAN AS REQUIRED.

#### DI			RESYNCHRONISE.

#### Orientation		SELECT REFERENCE POINT.

## EN-ROUTE/AIRFIELD APPROACH CHECKS.

#### Fuel			CHECK CONTENTS, SELECT APPROPRIATE TANK, MIXTURE LEANED.

#### Radios		TUNED, CORRECT FREQUENCY AND VOLUME.

#### Engine		T’s & P’S IN THE GREEN, CARB. HEAT FOR 10 SECONDS.

#### DI			RESYNCHRONISE.

#### Altimeter		SET QFE.

## PRE-STALLING/INCIPIENT SPINNING CHECKS

#### Height		SUFFICIENT TO RECOVER BY 2000’ agl DUAL.

#### Airframe		FLAPS AS REQUIRED.

#### Security		NO LOOSE ARTICLES, HATCHES CLOSED AND LATCHED, HARNESSES SECURE.

#### Engine		T’s & P’S IN THE GREEN, MIXTURE RICH, FULLER TANK SELECTED, FUELPUMP ON, CARB. HEAT FOR 10 SECONDS.

#### Location		CLEAR OF ACTIVE AIRFIELDS, BUILT-UP AREAS, CONTROLLED AIRSPACE.

#### Lookout		CLEAR OF OTHER AIRCRAFT AND CLOUD BOTH HORIZONTALLY AND VERTICALLY, CLEARING TURNS LEFT AND RIGHT THROUGH 90.

## AFTER STALLING CHECKS.

#### Fuel			CHECK CONTENTS, SELECT APPROPRIATE TANK, FUEL PUMP OFF.

#### Radio		TUNED AND CORRECT.

#### Engine		T’s & P’s IN THE GREEN.

#### DI			RESYNCHRONISE.

#### Altimeter		SET QNH.

## PRE-LANDING CHECKS.

#### Brakes		OFF.

#### Undercarriage		DOWN AND LOCKED.

#### Mixture		RICH, CARB. HEAT ON

#### Fuel			ON, SUFFICIENT FOR GO AROUND, FUEL PUMP ON, PRIMER IN AND LOCKED.

#### Instruments		T’s & P’s IN THE GREEN, SUCTION INDICATING, AMMETER READING NORMAL, DI SYNCHRONISED.

#### Carb. Heat		COLD

#### Hatches		CLOSED AND LATCHED.

#### Harnesses		SECURE.

## GO AROUND.

#### Throttle		FULL POWER.

#### Carb. Heat		COLD.

#### Airspeed		MINIMUM 55 KTS.

#### Flaps		DRAG FLAP UP.

#### Nose			SELECT CLIMB ATTITUDE.

## AFTER LANDING CHECKS.

#### Position		WELL CLEAR OF ACTIVE RUNWAY AND STOP.

#### Flaps		UP.

#### Throttle friction nut	LOOSE.

#### Trim			NEUTRAL.

#### Carb. Heat		COLD.

#### Fuel pump		OFF.

#### Landing light		OFF (EXCEPT NIGHTS).

#### Electrics		NON-ESSENTIAL LOADS OFF e.g. PITOT HEAT.

#### Fuel pump		OFF.

## SHUT DOWN CHECKS.

#### Position		INTO WIND, NOSE WHEEL STRAIGHT.

#### Parking brake		ON.

#### Throttle		SET 1200 RPM.

#### Radios		OFF.

#### Magnetos		CHECK DEAD CUT AND LIVE MAG.

#### Mixture		IDLE CUT OFF.

#### Throttle		FULL AFT TO AVOID ENGINE VIBRATION.

#### Magnetos		OFF WHEN ENGINE STOPS, KEY OUT.

#### Hobbs meter time	RECORD READING.

#### Master		OFF.

#### Fuel			OFF.

#### Seats		FULLY BACK.

#### Controls		SECURE WITH SAFETY BELT.

#### Cockpit		CLEAN AND HARNESSES TIDY FOR NEXT PILOT.

#### Tie downs		IN PLACE.

## MAXIMUM LANDING WEIGHT – 2 325 lbs.

#### Top speed		123 KIAS @ SL.

#### 75% cruise speed	117 KIAS @ 5000’.

#### 75% cruise speed	110 KIAS @ 2000’.

#### 65% cruise speed	103 KIAS @ 2000’.

#### Vr			52 KIAS.

#### Vx			63 KIAS.

#### Vy			79 KIAS.

#### Vfe			103 KIAS.

#### Cruise climb.		87 KIAS.

#### Demonstrated X/wind limit.	17 Kts.

#### Ground run – short field.	1200’.

#### Take off distance - 50’ obstacle. 1750’ (1/2 flap).

#### Take-off distance - short field. 2125’ (flaps up).

#### Landing roll - Short field.	590’.

#### Landing roll – 50’ obstacle.	1150’.

#### Tyre pressures main/nose	24 psi/30 psi

#### 29-Jul-26  input.doc