Preflight Checks

# Cockpit

| Control Wheel | Release Restraints |
| --- | --- |
| Parking Brake | Set |
| All Switches | OFF |
| Mixture | Idle Cut-Off (ICO) |
| Magnetos | Off & Key Out |
| Master Switch | On |
| Fuel Gauges | Check Quantity |
| Annunciator panel | Check |
| Cockpit lighting | Check |
| Beacon and Strobe lights (Nav. if night) | Check |
| Stall Warning vane | Check |
| Pitot Heat if flying below 5°C | Check |
| All Switches | Off |
| Master Switch | Off |
| Flaps | Check in stages, Set Down |
| Trim | Neutral |
| Pitot and Static Drains | Drain |
| First Aid Kit | In Position/Check |
| Fire Extinguisher | In Position, Secure |
| Windows | Check Clean |
| Required Papers | Check on Board |
| Tow Bar and Baggage | Stowed Correctly |
| Baggage Door | Closed and Secure |

# Right Wing

| Surface Condition | Clear of Ice, frost & Snow |
| --- | --- |
| Flaps and Hinges | Check |
| Aileron and Hinges | Check |
| Wing Tip and Lights | Check |
| Fuel Tank | Visual Check, Secure Cap |
| Fuel Tank Vent | Clear |
| Fuel Tank Sump | Drain |
| Tie Down and Chocks | Remove |
| Main oleo | Proper inflation ( 4.5 ±0.5 ) |
| Tyre | Check |
| Brake Block & Disc | Check |
| Fresh Air Inlet | Clear |

# Nose Section

| General Condition | Check |
| --- | --- |
| Cowling | Secure |
| Windshield | Clean |
| Propeller & Spinner | Check |
| Air Inlets | Clear |
| Chock | Remove |
| Nose oleo | Proper inflation ( 2.5 ±0.5 ) |
| Nose Wheel Tyre | Check |
| Engine Baffle Seals | Check |
| Oil | Check Quantity |
| Dip Stick | Properly Seated and orientated |
| Fuel Strainer | Drain |

# Left Wing

| Surface Condition | Clear of Ice, frost & Snow |
| --- | --- |
| Fresh Air Inlet | Clear |
| Chocks | Remove |
| Main oleo | Proper inflation ( 4.5 ±0.5 ) |
| Tyre | Check |
| Brake Block & Disk | Check |
| Fuel Tank | Visual Check, Secure Cap |
| Fuel Tank Vent | Clear |
| Fuel Tank Sump | Drain |
| Tie Down | Remove |
| Pitot Head | Check Holes |
| Wing Tip and Lights | Check |
| Aileron and Hinges | Check |
| Flaps and Hinges | Check |

# Fuselage

| Antennae | Check |
| --- | --- |
| Left Static pad | Clear |
| Empennage | Clear of Ice Frost, Snow |
| Fresh Air Inlet | Clear |
| Stabilator and Trim Tab | Check |
| Tie Down | Remove |
| Right Static vents | Clear |

Richard Wheeler 	T: 07801068013	www.gridline.co.uk

Internal & Starting

| Passengers | Board |
| --- | --- |
| Passengers | Safety Briefing |
| Cabin Door, Seats | Closed & Secure, Locked |
| Seat Belts & Harness | Fasten & Check Inertia Reel |
| Brakes | Set |
| Radios | Off |
| Instruments | Legible, Serviceable, Within Limits |
| Primary Flight Controls | Full and free and in correct sense |
| Trimmers | Set for take-off |
| Flaps | Raise |
| Cabin Air Controls | Closed |
| Throttle | Full and Free |
| Mixture | Full and free set rich |
| Throttle Friction | Loose |
| Propeller | Full and free set forward |
| Fuel Selector | On, Lowest tank |
| Alternate Air | Closed |

# Normal Start

| Mixture | Rich |
| --- | --- |
| Throttle | Full & Free, Set Half travel (E position) |
| Propeller | Fully forward |
| Master Switch | ON |
| Circuit Fuses | In |
| Lights/Beacons | On |
| Propeller | Clear |
| Shout | “Clear Prop” |
| Starter | Engage |
| Primer Button | Press as required |
| Throttle | Retard as Engine Starts 800RPM |

# External Power Start

| Master Switch | OFF |
| --- | --- |
| All Electrical Equipment | Off |
| Terminals | Connect |
| External Power Plug | Insert |
| Proceed with Normal Start |  |
| Throttle | Lowest Possible RPM |
| External Power Plug | Remove |
| Master Switch | On – Check Ammeter |

# Flooded Engine Start

| Mixture | Idle Cut Off |
| --- | --- |
| Throttle | Full & Free, Set Full Forward |
| Propeller | Forward |
| Master Switch | ON |
| Auxiliary Fuel Pump | OFF |
| Propeller | Clear |
| Starter | Engage |
| When Engine Fires | Prime button press as required. |
| Throttle | Retard as engine starts |
| Mixture | Advance slowly |

# Cold Weather Start

| Fuel Selector | ON |
| --- | --- |
| Mixture | Idle Cut-Off |
| Throttle | Full Forward |
| Propeller | Forward |
| Master Switch | ON |
| Auxiliary Fuel Pump | High Boost |
| Propeller | Clear |
| Starter | Engage |
| Mixture | Full Forward for 3 Seconds |
| Throttle | Full Forward to Full aft |
| Mixture | Full Forward 3 Secs, Full Aft 3 Seconds Repeat |
| When Engine Fires |  |
| Starter | Leave Engaged |
| Auxiliary Fuel Pump | Low Boost |
| Primer Button | Tap, until Rhythmic firing |
| Starter | Release |
| Throttle | Half travel |
| Oil Pressure | Check |
| If Engine Faulters |  |
| Primer Button | Tap |
| Throttle | 1000 rpm |
| Auxiliary Fuel Pump | Off when Starts |

Richard Wheeler 	T: 07801068013	www.gridline.co.uk

### After Start

| Starter warning light (LHS) | Out |
| --- | --- |
| Oil Pressure | Rising |
| Throttle | Set 1300 RPM |
| Alternator | Check positive charge |
| Gyro Vacuum | Check more than 3 inches Hg |
| Radio Navaids | Set and tested |
| Flight Instruments | Check & Set |
| Magnetos | Check |
| RT | Request taxi for ….VFR flight |
| Altimeter | Set |
| Throttle | 900-1200 RPM |

# Taxiing

| Throttle | Apply Slowly |
| --- | --- |
| Prop | Fully forward |
| Brakes | Check |
| Steering | Check full |
| Instruments | Check during turns |

# Power Checks

| Face Into Wind |  |
| --- | --- |
| Brakes | Parking brake on |
| Fuel Tank | Change to fullest tank |
| T’s & P’s | In the green |
| Propeller | High RPM |
| Throttle | Set 2000 RPM |
| Magnetos | Max. drop 150, max. differential 50 RPM |
| Vacuum | 4.8-5.1 inches Hg |
| Oil T’s & P’s | Check |
| Annunciator Panel | Test |
| Propeller | Cycle 3 times (keep above 1500RPM), Set fine |
| Alternate Air | Check, Off |
| Auxiliary Fuel Pump | Off |
| Throttle | Close, Check 600-800RPM, Return to 1300 |
| Oil Temperature | 100ºF for Take-off |

## Before Take Off

| Flight Instruments | Check |
| --- | --- |
| Fuel Tank | Fuller Tank |
| Booster pump | Off |
| Alternate Air | Close |
| Seat Backs | Erect |
| Mixture | Rich |
| Propeller lever | Fully forward |
| Throttle Friction | Finger Tight |
| Belts/Harnesses | Secure |
| Empty Seats | Seat Belts Snugly Fastened |
| Flaps | 2nd Stage, 25º, for grass take-off |
| T’s & P’s | In the green |
| Trim Tab | Set take-off |
| Controls | Full and Free |
| Gyros, DI and compass | Synchronised |
| Auto Pilot | Off, breaker identified |
| Door | Closed and latched |
| Crew briefing | Confirm who is controlling pilot in the event of an emergency |

Richard Wheeler	T: 07801068013	www.gridline.co.uk

## Flight

# Normal Take-off

| Flaps | Set 1st Stage 10º |
| --- | --- |
| Tab | Set |
| Accelerate to 71-77KIAS |  |
| Control Wheel | Back Pressure |

# Short Field Take Off

| Flaps | Set 2nd Stage, 25º |
| --- | --- |
| Accelerate to 55-58KIAS, Depending on Weight |  |
| Control Wheel | Back Pressure |
| Accelerate to 60-66KIAS after breaking ground |  |
| Accelerate to 75 KIAS for Best Angle of Climb |  |
| Flaps raise at 200’ | Slowly Reduce |
| Accelerate to 90 KIAS for Best Rate of Climb |  |

# Soft Field Take Off

| Flaps | Set 2nd Stage, 25º |
| --- | --- |
| Accelerate to 55-58KIAS, Depending on Weight |  |
| Control Wheel | Back Pressure |
| Accelerate to 60-66KIAS after breaking ground |  |
| Accelerate to 90 KIAS for Best Rate of Climb |  |
| Flaps | Slowly Reduce |

# Take-off Climb

| Mixture | Full Rich |
| --- | --- |
| Prop Speed | 2575 RPM |
| Manifold Pressure | Do not exceed 41 inches Hg |
| Climb Speed ( Best Angle ) | 75 KIAS |
| Climb Speed ( Best Rate ) | 90 KIAS |
| Auxiliary Fuel Pump | LO – If Required |

# Cruise Climb

| Mixture | Full Rich |
| --- | --- |
| Prop Speed | 2450 RPM |
| Manifold Pressure | 33 inches Hg |
| Climb Speed | 104 KIAS |
| Auxiliary Fuel Pump | LO – If Required |

# Cruising

| Normal Max Power | 75% |
| --- | --- |
| Mixture | Adjust |
| Auxiliary Fuel Pump | LO – If Required |

# Approach and Landing

| Brakes | Off |
| --- | --- |
| Mixture | Set Rich |
| Propeller | Set 2600RPM |
| Fuel Selector | Proper Tank |
| Flaps | 102 set-KIAS max |
| T’s & P’s | Check |
| Seat Backs | Erect |
| Harnesses & Hatches | Locked/Secured |
| Auto Pilot | Off |
| Trim to 74 KIAS |  |

# Stopping Engine

| Flaps | Retract |
| --- | --- |
| Radios | Off |
| Propeller | Full Increase |
| Throttle | Full Aft |
| Mixture | Idle Cut-Off |
| Magnetos | Off |
| Master Switch | Off |

# Parking

| Parking Brake | Set |
| --- | --- |
| Control Wheel | Secured with Belts |
| Flaps | Full Up |
| Wheel Chocks | In Place |
| Tue Downs | Secure |

# Pre-stall checks

| H : Height | Sufficient to recover by 2500ft agl. dual |
| --- | --- |
| A : Airframe | Flaps as required |
| S : Security | Hatches closed and harnesses secure, no loose articles |
| E : Engine | Mixture rich, Fullest tank selected, T’s & P’s in the green |
| L : Location | Clear of Airfields, built up areas, cloud and controlled airspace |
| L : Lookout | Turns 90° left and right |

## Emergency

# Fire On The Ground

| Throttle | Close |
| --- | --- |
| Mixture | Fully lean |
| Fuel | Off, Pump Off |
| Magnetos | Off |
| Master Switch | Off |
| Parking Brake | On |

# Cabin Fire in The Air

| Master Switch | Off if electrical Fire |
| --- | --- |
| Electrical Circuits | Off as Required |
| Fire Extinguisher | Use only if Absolutely Necessary |

# Engine Fire In The Air

| Throttle | Close |
| --- | --- |
| Mixture | Fully lean |
| Fuel | Off, Pump Off |
| Magnetos | Off |
| Cabin Heaters/Defrost | Off |

# Engine Failure on Take-off

| Fuel | Off, Pump Off |
| --- | --- |
| Magnetos | Off |
| Master Switch | Off |

Richard Wheeler 	T: 07801068013	www.gridline.co.uk

# Engine Failure at Altitude

| Attain & Maintain Best Glide Speed |  |
| --- | --- |
| Assess Surface Wind |  |
| Select Suitable Landing Area |  |
| Plan Approach Pattern |  |

# Restart Engine

| Fuel | Change Tanks |
| --- | --- |
| Mixture | Check |
| Magnetos | Check L & R, Both |
| Throttle | Check |

# Committed Checks

| Fuel | Off |
| --- | --- |
| Magnetos | Off |
| Harness | Tight |
| Doors | Unlatched |
| Crew/Passengers | Briefed |
| RT | Mayday Call |
| Master Switch | Off |

### Ditching Procedure

| Radio | Mayday call |
| --- | --- |
| Transponder | 7700 |
| Harness | Tight |
| Doors | Unlatched |
| Crew/Passengers | Briefed |
| Large Swell/Light Wind Light Swell/Strong Wind | Land along swell Land into Wind |