G-BNYB Checklist

Cockpit

Control Wheel	     Release Restraints

Parking Brake		Set

All Switches		OFF

Mixture			Idle Cut-Off

Magnetos		Off & Key Out

Master Switch		On

Fuel Gauges		Check Quantity

Annunciator Panel		Check

Cockpit Lighting		Check

Nav & Strobe Lights 	Check

Stall Warning		Check

Pitot Heat		Check

All Switches		OFF

Master Switch		OFF

Flaps		    Check in Stages			Set Down 2nd  Stage 25o

Trim			Neutral

Pitot &Static Drains 	 Drain

First Aid Kit		In Position/Check

Fire Extinguisher		In Position, Secure

Windows			Check Clean

Required Papers		Check on Board

Tow and Baggage		Stowed Correctly

Baggage Door		Closed & Secure

Right Wing

Surface Condition	     Clear of Ice, Frost & Snow

Flaps & Hinges		Check

Aileron & Hinges		Check

Wing Tip & Lights		Check

Fuel Tank	     Visual Check, Secure Cap

Fuel Tank Vent		Clear

Fuel Tank Sump		Drain

Tie Down & Chocks	 Remove

Main Gear Strut	      Proper Inflation (4.5 ±0.5)

Tyre			Check

Brake Block & Dis		Check

Fresh Air Inlet		Clear

Nose Section

General Condition		Check

Cowling			Secure

Windshield		Clean

Propeller & Spinner	 Check

Air Inlets			Clear

Alternator Belt		Check Tension

Chock			Remove

Nose Gear Strut	      Proper Inflation (2.5 ±0.5)

Nose Wheel Tyre		Check

Engine Baffle Seals              Check

Oil			Check Quantity

Dip Stick			Properly Seated

Fuel Strainer		Drain

Left Wing

Surface Condition	     Clear of Ice, Frost & Snow

Flaps & Hinges		Check

Aileron & Hinges		Check

Wing Tip & Lights		Check

Fuel Tank	    Visual Check, Secure Cap

Fuel Tank Vent		Clear

Fuel Tank Sump		Drain

Tie Down & Chocks 	Remove

Main Gear Strut	      Proper Inflation (4.5 ±0.5)

Tyre			Check

Brake Block & Disc 	Check

Fresh Air Inlet		Clear

Pitot Head		Check Holes

Fuselage

Antennas			Check

Left Static Pad		Clear

Empennage	        Clear of Ice Frost, Snow

Fresh Air Inlet		Clear

Stabilator and Trim Tab   	Check

Tie Down			Remove

Right Static Pad		Clear

Internal & Starting

Passengers		Board

Crew/Passengers		Safety Briefing

Cabin Door, Seats	Close & Secure, Locked

S. Belts & Harness Fasten & Check Inertia Reel

Brakes 			Set

Radios			OFF

Instruments Legible     Serviceable, Within Limits

Primary Flight Controls       	 Proper Operation

Trimmers		Set for Take Off

Flaps		Set Up

Cabin Air Controls		Closed

Throttle		Full & Free

Mixture		Full & Free

Throttle Friction		Loose

Propeller	       Full INCREASE rpm

Fuel Selector		On, Lowest Tank

Alternate Air		Closed

Normal Start

Mixture		Rich

Throttle	Full & Free, Set Half Travel

Propeller		Forward

Master Switch		ON

Circuit Fuses		In

Lights/Beacons		On

Propeller		Clear

Shout		“Clear Prop”

Starter			 Engage

Primer Button		On As Required

Throttle	         Retard as Engine Starts 800RPM

External Power Start

Master Switch		          OFF

All Electrical Equipment	          OFF

Terminals		         Connect

External Power Plug	          Insert

Proceed with Normal Start

External Power Plug 	          Remove

Master Switch 	              ON-Check Ammeter

Flooded Engine Start

Mixture		   Idle Cut Off

Throttle		   Full & Free, Set Forward

Propeller		   Forward

Master Switch	   ON

Auxiliary Fuel Pump  OFF

Propeller		   Clear

Starter		   Engage

When Engine Fires   Prime as required

Throttle		   Retard as Engine Starts

Mixture		   Advance Slowly

Cold Weather Start

Fuel Selector	ON

Mixture		Idle Cut-Off

Throttle		Full Forward

Propeller		Forward

Master Switch 	ON

Auxiliary Pump	High Boost

Propeller		Forward /  Clear

Starter		Engage

Mixture 		Full Forward for 3 Seconds

Throttle		Full Forward to Full Aft

Mixture Full Forward 3 Secs, Full Aft 3 Secs

Repeat

When Engine Fires

Starter		Leave Engaged

Auxiliary Fuel Pump  Low Boost

Primer Button         Tap until Rhythmically Firing

Starter		Release

Throttle		Half Travel

Oil Pressure	Check

If Engine Falters

Primer Button	Tap

Throttle		1000 rpm

Auxiliary Fuel Pump  OFF When Starts

After Start

Starter Warning Light Out

Oil Pressure	Check

Throttle		Set 1300 rpm

Alternator	Check Positive Charge

Gyro Vacuum	Check more than 3

Radio Navs	ON

Radio Aids	Check

Flight Instruments	Check & Set

Magnetos	Check

RT		Call for Clearance

Altimeter		Set

Throttle 		900-1200 rpm

Taxiing

Throttle			Apply Slowly

Prop			Lever Full Forward

Brakes			Check

Steering			Check Full & Free

Instruments		Check / Turn

Power Checks

Face Into Wind

Brakes			ON

Fuel Tank		Change Tanks

Temperature & Pressures  	Green

Propeller			Full Increase

Throttle			2000 rpm

Magnetos		Max Drop 150

Max Diff 50 rpm

Vacuum			4.8-5.1 Hg

Oil Temperature & Pressures   Check

Annunciator Panel	  	Check

Propeller	                       Down to 1500 rpm

Exercise    3 times, Full Increase

Alternate Air		Check, OFF

Auxiliary Fuel Pump	 OFF

Throttle Close  Check     600rpm  Return to 1300

Oil Temperature 	         100ºF for Take Off

Fuel Pressure 		Check

Throttle 			Retard

Manifold Pressure Line  	Drain

Before Take Off

Flight Instruments 		Check

Fuel Tank 		Fuller Tank

Auxiliary Pump		OFF

Alternate Air		Close

Seat Backs		Erect

Mixture			Set Rich

Prop			Set MAX rpm

Throttle Friction		Finger Tight

Belts/Harness		Fastened

Empty Seats	  Seat Belts Snugly Fastened

Flaps			Set for Take Off

T’s & Pressures  		Check

Trim Tab			Set

Controls			Full & Free

Gyros, DI			Set

Auto Pilot		OFF

Doors			Latched

P.T.O.

Normal Take-off Normal Take Off

Flaps			Set 1st  Stage 10º

Tab			Set

Accelerate to 		71-77KIAS

Control Wheel		Back Pressure

Short Field Take-off

Flaps			Set 2nd Stage, 25º

Accelerate to        55-58,    Depending on weight

Control Wheel		Back Pressure

Accelerate to  60-66 KIAS  after breaking ground

Accelerate to  75 KIAS for   Best Angle of Climb

Flaps 			Slowly Reduce

Accelerate to  90 KIAS for    Best Rate of Climb

Soft Field Take-off

Flaps			Set 2nd Stage, 25º

Accelerate to       55-58    Depending on weight

Control Wheel		Back Pressure

Accelerate to 60-66 KIAS after breaking ground

Accelerate to 90 KIAS   for   Best Rate of Climb

Flaps 			Slowly Reduce

Take-off Climb

Mixture 			Full Rich

Prop Speed		2575 rpm

Manifold Pressure	        Do Not Exceed 41 in Hg

Climb Speed (Best Angle) 	75 KIAS

Climb Speed (Best Rate)   	90 KIAS

Auxiliary Fuel Pump       LO- If Required

Cruise Climb

Mixture			Full Rich

Prop Speed		2450

Manifold Pressure		33 in Hg

Climb Speed 		104 KIAS

Auxiliary Fuel Pump	 LO-If Required

Cruising

Normal Max Power	75%

Mixture			Adjust

Auxiliary Fuel Pump 	 LO- If Required

Approach & Landing

Brakes			Brakes Off

Mixture			Set Rich

Propeller			Set 2600 rpm

Fuel Selector		Proper Tank

Flaps			102 KIAS Max

T’s & P’s			Check

Seat Backs		Erect

Hatches & Harnesses	 Locked/Secured

Auto Pilot		Off

Trim to 74 KIAS

Stopping Engine

Flaps			Retract

Radios			Off

Magnetos		Check & Off

Propeller		Full Increase

Throttle 			Full Aft

Mixture 			Idle Cut-Off

Master Switch 		Off

Parking on Field

Parking Brake		Set

Control Wheel		Secured with Belts

Flaps			Full Up

Wheel Chock		In Place

Tie Downs 			Secure

Parking for Hangar

Brakes			Not Applied

Control Wheel		Not Secured

Keys to Tower Personnel

Pre-Landing Checks / Downwind

B 	Brakes		 Off

U 	Undercarriage	 Fixed Down

M	Mixture Rich

F 	Fuel on and Sufficient for

Go-Around
I  	Instruments T’s & P’s Alt. Set
C 	Carb. Heat Check & Set as Required
H	Hatches Secure
H	Harnesses Secure & Fastened

Cruise / Rejoin Checks

F    Fuel	       On & Sufficient Mixture Rich
 R    Radio       Correct Frequency Make

Any Necessary calls
 E    Engine     T’s & P’s & other Instruments	
 D    Direction   DI Synchronised with compass

(In Straight And Level Flight) 
 A   Altitude    Check & Sub-Scale Correct

Pre-Stall Aerobatic Checks

H  Height  Sufficient to recover by 3000ft

A  Airframe   	Flaps as Required

S  Security            Hatches & Harnesses No Loose Articles

E  Engine  	Mixture Rich, Fuller Tank, T’s & P’s

L   Location	Clear of Airfields, Built Up Areas, Cloud & Controlled Airspace

L   Lookout 	Check

Station                   Frequency

Bembridge Radio	123.25

Blackbushe			122.30

Boscombe Radar	126.70

Bournemouth App.	119.475

Bournemouth  Twr	125.60

Farnborough App.	125.25

Gatwick			126.825

Goodwood Info.		122.45

Lashenden / Headcorn	122.00

Lydd			120.70

Popham			129.80

Sandown			119.275

Shoreham App		123.15

Solent App.			120.225

Wycombe Tower		126.55

Goodwood     DME           114.75

London Info.		124.75 or

124.60

Mayday			121.5

MAYDAY

If Time Permits But Fly The Plane

Mayday  Mayday  Mayday

Engine Failure In Flight

Establish Best Glide Speed 	95 KIAS

Select Landing Area Into Wind

Plan Approach Pattern

Check for Causes Of Failure

Fuel Pump 		On

Fuel Selector	Change Tanks

Mixture 		Rich

Magnetos		Check

Throttle		Check

Attempt Re-start	If Unsuccessful

Committed Checks

Brakes 		Off

Mixture		Idle Cut-Off

Fuel Selector	Off

Harnesses		Tight

Passenger Brief	Brace, Brace, Brace.

STATION

G-BNYB is a Piper Dakota Aircraft

Nature of Emergency

Intention

Present or Last Known Position

Flight Level / Altitude

Heading

Pilot Qualifications

Any Useful Information

People on Board

Emergency

Fire On The Ground

Throttle		Close

Mixture		Fully Lean

Fuel		Off, Pump Off

Magnetos	Off

Master Switch	Off

Parking Brake 	On

Cabin Fire In The Air

Master Switch 	Off If Electrical Fire

Electrical Circuits	Off As Required

Fire Extinguisher	Use only if Absolutely Necessary

Engine Fire In The Air

Throttle		Close

Mixture		Fully Lean

Fuel		Off, Pump Off

Magnetos	Off

Cabin Heaters/Defrost    Off

Engine Failure On Take-Off

Fuel	Off, Pump Off

Magnetos	Off

Master Switch	Off