- It is the sole responsibility of the pilot in command to brief him- or herself fully prior to the flight.

- The aircraft must carry a serviceable radio. Pilots must make blind transmissions addressed to ‘Goodwood Traffic’ on 122.455 whilst on the ground and within the ATZ, broadcasting their positions and intentions.

- Use the runway aligned closest into wind.  If a larger aircraft has to use a different runway, others should clear the circuit until any such operation is completed.

- Join the circuit using the standard overhead join procedure from 2000ft QFE.  If the cloud base prevents this, broadcast your position and intentions clearly.

- On arrival or departure, pilots must fly the standard circuit pattern for the relevant runway.

- Pilots of departing aircraft must familiarize themselves with the noise abatement routing and fly the route for the relevant runway accurately.

- Training circuits i.e. touch and goes or low approach and go-arounds and practice forced landings, are not permitted Out of Hours.

- Pilots must be aware that birds may congregate on the runways or manoeuvring area or be flying in the vicinity of the airfield.

- After completion of the movement(s), the PIC or operator must advise Goodwood ATS by email or telephone of runway used, the departure point/destination and the actual times of arrival and/or departure by 1000L the following day.