# IFR Radio Navigation Equipment when in Controlled Airspace

2 	Scale of Equipment Required – A, E1 and F

Aircraft and Circumstances of flight

- All aircraft (other than gliders) within the United Kingdom:

- when flying under Instrument Flight Rules within controlled airspace

- when flying within controlled airspace

- when making an approach to landing at an aerodrome notified for the purpose of this sub-paragraph

3 	(2) An aircraft which is not a public transport aircraft and which is flying in Class D or Class E airspace shall not be required to be provided with distance measuring equipment in accordance with paragraph (b) of Scale F when flying in the circumstances specified in sub-paragraph (1)(a) of paragraph 2.

4 	The scales of radio and radio navigation equipment indicated in the foregoing Table shall be as follows:

Scale A

Radio equipment capable of maintaining direct two-way communication with the appropriate aeronautical radio stations.

Scale E1

Secondary surveillance radar equipment which includes a pressure altitude reporting transponder capable of operating in Mode A and Mode C and is capable of being operated in accordance with such instructions as may be given to the aircraft by the air traffic control unit.

Scale F

Radio communication and radio navigation equipment capable of enabling the aircraft to be navigated along the intended route including:

(a) automatic direction finding equipment;

(b) distance measuring equipment and

(c) VHF omni-range equipment.

Scale G

Radio navigation equipment capable of enabling the aircraft to make an approach to landing using the Instrument Landing System.