# PATHFINDER FLYING CLUB (JUN 05)

Robin DR400/180

CHECKLIST

    - INITIAL CHECKS

    - EXTERNAL CHECKS

    - PRE-START CHECKS

    - START CHECKS

    - AFTER START CHECKS

    - TAXY CHECKS

    - RUN-UP/POWER CHECKS

    - PRE TAKE-OFF CHECKS VITAL ACTIONS

    - RUNWAY CHECKS

    - CHECKS DURING TAKE-OFF

    - AFTER TAKE-OFF CHECKS

    - AIRFIELD DEPARTURE CHECKS

    - CRUISE & DESCENT/REJOIN CHECKS

    - ENTERING LOW LEVEL CHECKS

    - STALLING CHECKS

    - PRE LANDING CHECKS

    - FINALS CHECKS

    - AFTER LANDING CHECKS

    - SHUTDOWN CHECKS

    - BASIC DATA

Max Gross Wt 		2425 lbs (1100kg)

Max Fuel Wt 	  299 lbs (137 kg)

Whilst this flight guide has been produced with reference to the CAA approved aircraft flight manual, it is not subject to amendment and its use does not absolve pilots from operating the aircraft iaw the flight manual.

1.	INITIAL CHECKS

On approaching the aircraft check:

General position		Safe location to taxy

Ground fire extinguisher	Available

Airframe			Free from ice, contamination

Before commencing the external checks carry out the following in the cockpit:

Parking brake 		ON

Fire extinguisher		Check, secure

First aid kit			Secure

Headsets			Available

Documents			Stowed

Baggage area		Loose articles secured

(max 60 kg)

Flaps				Check operation

Magneto switch		OFF, key out

Battery Switch		ON

Pitot heater			ON /check/off

Landing/taxy lights	ON /check/off

Navigation lights		ON /check/off

Strobe lights		ON /check/off

Stall warning vane		Check operation

+ audible warning

Fuel contents gauge	Check contents and confirm with tech log and visual check of wing tanks

Battery switch 		OFF

Fuel Cock 	ON

2.	EXTERNAL CHECKS

Start at left wing inboard edge.

# Left Wing

Fuel filler cap 	secure

Flap	Condition, play, linkage, hinges

Undercarriage		Tyre, brake leaks, spat secure

Aileron	Condition, play, drains, hinges

Wing surfaces		Condition, drains

Inspection Cover		Security

Wingtip			Nav light

Leading Edge		Condition

Landing/taxy Light		Undamaged

Pitot Head			Remove cover/hole clear

Undercarriage	Condition, extension, tyre condition/creep, inflation. Brakes – damage and leaks, spat secure

Flap Underside	Condition, drains clear

Wing tank fuel drain	Check for water contamination

## Forward Fuselage

Fuel drain	Check for water contamination (fuel selector on)

Canopy emergency

Release - port	Wire tell-tale secure

Canopy Runner	Secure and undamaged

Cowling Port Side	Security, 3 fasteners, 1 lug with safety pin, 1 screw, no oil leaks

Fresh Air Intake	Clear

Propeller	Condition, spinner secure

Nosewheel	Condition, extension, tyre-cuts/creep/inflation, spat secure

Ram Air Inlet	Check clear

Cowling Stbd Side	Security, 3 fasteners, 1 lug with safety pin, 1 screw, oil leaks

Oil Contents-minimum 6 US quarts, panel secure (do not over tighten dipstick)

Canopy Runner	Secure and undamaged

Canopy emergency

Release - stbd		Wire tell-tale secure

# Right Wing

Fuel filler cap 	secure

Wing tank fuel drain	Check for water contamination

Leading Edge		Condition

Undercarriage	Condition, extension, tyre condition/creep, inflation. Brakes – damage and leaks, spat secure

Flap Underside		Condition

Wing surfaces		Condition, drains

Inspection Cover		Security

Wingtip			Nav light

Aileron	Condition, play, drains

Wing				Drains

Undercarriage	Tyre, brake leaks, spat secure

Flap				Condition, play, linkage,					hinges

## Rear Fuselage

Static Vent - Stbd	Clean, unobstructed

Fin fairing	Secure

Elevator	Secure, hinge clearance

Strobe Light	Condition

Rudder	DO NOT MOVE. Condition, stiff nut, nav lts, hinge clearance.

Trim Tab	Position, stiff nut, security, play, hinges

Static Vent - Port	Clean, unobstructed

Baggage Door	Secured [max 60 kg]

3.	PRESTART CHECKS

Passenger Briefing	Stated

Harness	Secure (3 straps) Solo, secure RH harness, rear seat harnesses

Front Seats	Adjusted and locked

Headset			Plugged in, don

Master Switch		ON, intercom set & check

Circuit Breakers		All made

CO Monitor			Normal

Ammeter			Condition

Alternator			OFF

Throttle			Free movement – Set closed

Air Vents			Closed

Flight Instruments		Check Condition

Warning Lights		Test

Radios			OFF

Nav Equipment 		OFF

Transponder		OFF

Cabin Heat			OFF

Pitot Heat			OFF

Instrument Lights	OFF (except night flying)

External Lights	OFF (except nav lights for night flying)

Carburettor Heat		Full movement and Cold

Engine instruments	Condition

Tacho Time (RPM gauge) Note reading

Fuel Boost pump	OFF

Mixture	Full movement and Rich (Up)

Fuel Cock			ON, Check contents – all tanks,

Select left tank

Parking Brake		Set ON

Trim	Full free movement and set Neutral

Flaps				Up

Controls	Full free movement – correct sense (except rudder) – no play or excessive friction

Canopy			Closed and locked

4.	START CHECKS

Strobe Light			ON

Carb Heat 			OFF

Fuel Pump			ON - check fuel pressure rises

Mixture			Full rich

Key In			Mags off

Throttle	Prime as required(slowly pump throttle 4 times if cold, none if CHT  250’F), set 1/4” open

Magneto			Left

Intercom			OFF

Propeller	Clear -call “Clear Prop”

Intercom			ON

Starter	Press, release when engine fires (max 30 secs)

Starter Engaged Light	Out (If not, shutdown and investigate)

Magnetos			Both

Throttle			Set 1200 rpm

Oil Pressure			Rising within 30 seconds

### Alternator 			ON

5.	AFTER START CHECKS

Fuel Pump			OFF

Fuel cock			Select right tank

Alternator			Charging

Magneto			Check for dead magneto

Suction			Indicating

Horizon			Erecting, adjust datum

DI				Synchronise

Radios			ON, frequencies set

Nav Equipment		ON, frequencies set

Transponder		Standby, 7000 set

Radio				RT check & taxy clearance

Landing Light		ON (night flying only)

- TAXY CHECKS

Brakes	Check immediately (dual, both sides)

Flight Instruments	Check turn co-ordinator, slip ball, compass, DI, AI

Right turn: needle right, ball left, numbers increasing, AI steady

Left turn: needle left, ball right, numbers decreasing, AI steady

Rudder			Check full and free movement

7.	RUN UP/POWER CHECKS

#### Park aircraft into wind with the nosewheel straight

Fuel cock 			select main tank

Parking Brake		ON

Safety	Canopy locked, controls central, clear behind

Oil Pressure	 & temp	Green range

Fuel Pressure		Green range

Mixture 			Fully rich

Carb heat			Cold

Throttle	Set 2000 rpm - check brakes holding

Suction			Indicating – Green range

Ammeter			Positive charge indicating

Carburetor Heat	Select Hot - Check decrease in rpm then reselect Cold

Magnetos	Check Left & Right in turn. Max drop 175 rpm, max 50 rpm difference between Left & Right

Mixture	Lean until RPM reduction then reselect full rich

Throttle	Close. Check smooth idle 600 – 650.  Reset 1200 rpm

8.	PRE TAKE-OFF CHECKS (VITAL ACTIONS)

Pitot Heater			As required

Suction			Check

TC Flag			Clear

Flight Instruments		Check and set – AI, DI, TC

Engine Ts and Ps		Check

Transponder	Test (check light on) set to ALT

Carburetor Heat		Cold

Mixture 			Full rich

Magnetos			Both

Fuel Cock	ON, contents sufficient (Note:  Main tank is recommended for all circuit flying)

Fuel Pump			ON, check pressure increase

Flaps	Take-off - check lift off speed

(- 54 KIAS)

Elevator Trim		Set at Neutral

Seat Belt			Tight and secure

Controls (Ail/Elev)		Full & free movement

Canopy			Closed & locked

TAKE-OFF BRIEF:

The following points must be briefed:

i.	Operating pilot for the departure

ii.	Runway length and crosswind

iii.	Action in the event of an engine failure on the ground

iv.	Action in the event of an eng fail after take-off

- Action of non-operating pilot/passenger

Radio	Call 				As required

9.	RUNWAY CHECKS

Take-off Time			Note

Approach & Departure lanes	Clear

Compass/DI/Rwy Heading	Aligned

10.	CHECKS DURING TAKE-OFF

Throttle	Full pwr, min 2200 rpm

Engine Temps & Pressures	Check ASI Increasing

Take-off speed (take-off flap)	54 kias

11.	AFTER TAKE-OFF CHECKS

Climb speed (2425 lb)

Flaps at take-off

- best rate 81 kias, best angle 70 kias

Flaps up

- best rate 92 kias, best angle 76 kias

Engine Temps & pressures	Check

Flaps					Raise A/R

12.	AIRFIELD DEPARTURE CHECKS

Fuel Pump				OFF (at safe height)

Landing Light			OFF

Radio/Nav Equipment		Set (obtain FIS/RIS/RAS)

Altimeter	Set (Note airfield QNH/QFE if returning)

13.	CRUISE, PERIODIC & DESCENT (REJOIN) CHECKS

Fuel				Fuel pumps as required

Fuel cock as required

Throttle set as required

Mixture set as required

Contents & pressure checked

Fuel tank selection

Radios/Navaids		Set as required

Transponder - ALT

Ammeter charging

Circuit breakers in

Engine			Ts & Ps checked

Carb ice check

CO Monitor - Normal

Direction Indicator		Align with compass

Check suction

Altimeter			Set as required

14.	ENTERING LOW LEVEL

Before descending carry out FREDA checks, then:

Harnesses			Secure

External Lights		All ON

Lookout			Clear entry area

15.	STALLING CHECKS

Height	Sufficient to recover by briefed height

Airframe	Flaps up – as required

No mist/ice on canopy

Security			Harnesses secure

Canopy locked

Loose articles stowed

Engine			Fuel pump ON

Fuel cock – Main tank

Mixture rich

Fuel contents & pressure checked

Ts & Ps checked

Carb ice check

Location			Clear of:

Active airfields

Built up areas

Controlled airspace & cloud

Danger /Restricted areas

Good horizon available

Lookout	Clear above and below. Min 180 before first stall, then 90

Stalling speeds:

- Flaps up			57 kias

- Flaps take-off		53 kias

- Flaps land			51 kias

16.	PRE-LANDING CHECKS

Brakes			OFF

Undercarriage		Down & Locked.

Mixture			Fully Rich

Fuel cock	ON, contents sufficient. (Note:  Main tank is recommended for all circuit flying)

Fuel pump			ON

Indicators			Ts & Ps checked

Carb Heat			Hot

Hatches			Secure

Harnesses			Secure

Flap				Take-off (max 92 kias)

17.	FINALS CHECKS

Clearance			Obtained

Carburetor Heat		Cold

Flaps 				As required

Brakes			Toes Clear

18.	CROSSWIND/GUSTING LANDING

Flaps				Take-off

App Speed			70 kias + ½ wind gusts

Correct for drift

Demonstrated crosswind 22 kias

19.	GO AROUND

Carb heat			Cold

Throttle 			Open Fully

Speed			67 kias

When safe to do so:

Flaps				Take-off

Speed			78 kias

20.	AFTER LANDING CHECKS

Landing time		Note

Pitot Heat			OFF

External Lights	As required (Strobe light ON)

Nav Equipment		OFF

Transponder		OFF

Carb Heat			Cold

Fuel pump			OFF

Flaps				UP

Trim				Set neutral

Fuel required ?

21.	SHUTDOWN CHECKS

Parking Brake		On

Throttle			Set 1200 rpm

Radios			Off

Magnetos			Dead Cut check

Throttle			Set 1000 rpm

Mixture			Idle Cut Off

### Alternator			Off

When propeller has stopped:

Magnetos			Off, key out

Fuel Cock			Off

External Lights		Off

Master Switch		Off

Flaps				Down

Tacho Time 			Note reading

Headsets			Remove

Harness			Release, loosen & tidy straps

Aircraft	Vacate - remove personal belongings

Chocks?

# ROBIN DR400-180 – BASIC DATA

1.  GENERAL CONSTRUCTION

The Robin DR400-180 Regent is a four-seat, single engine, low-wing monoplane of wooden construction.

2.  REGISTRATION CATEGORY & PERFOMANCE GROUP

Aircraft classified as -	Aeroplane (Landplane)

Aircraft classified in -	Performance Group ‘E’

Aircraft certificated in -	Public Transport Category

3.  FLIGHT CONDITION LIMITATIONS

Flight in known or

forecast icing conditions		-	Not cleared

Flight at night 			-	Cleared*

Flight in instrument

Meteorological Conditions	-	Cleared*

* Flight permitted subject to carriage of appropriate equipment

4.  DIMENSIONS

Length			:	  7.10 m

Wingspan			:	  8.72 m

Height			:	  2.23 m

5.  ENGINE

Engine type			:	Avco Lycoming 0-360-A3, 4   cylinder, 4 stroke

##### Engine rating			: 	180 HP at 2700 RPM

##### Max permitted RPM		: 	2700

Avoid cont ops between 2150 & 2350 rpm

Cylinder head

temp range		: 	0o to 230o (Green arc)

Propeller type		: 	Sensenich76EM8S5-0-64

Propeller diameter		: 	1.93 m

Propeller pitch		: 	Fixed (64”)

Associated Engine controls / indicators:

RPM gauge, throttle, magneto switch, starter button, starter engaged light, oil pressure gauge, oil temperature gauge, cylinder head temperature gauge.

6.   ENGINE LUBRICATION

Oil type (all temps)		:	SAE 15W-50 or 20W-50

Oil capacity (max/min)	:	8 US qts (club min 6 )

Oil consumption

(cruise power)	:	Approx 4 hrs per US qt

Oil circulation	    	:       Wetsump, Engine-driven pump

Oil temperature 			Max 118o C

(normal range)		:	60o – 118o C (green arc)

Oil pressure 			Min 25 psi (at idle)

(normal range)		:	55- 95 psi (green arc)

Max 115 psi

Oil pressure must be in green arc within 30 seconds of engine start.

7.   FUEL SYSTEM

Fuel type		:	AVGAS 100 LL

Fuel pump	     :	Engine driven + electric booster

Fuel capacity (total)	     : 	41.8 Imp Gals (190 Litres)

Fuel capacity (usable)     :	41.58 Imp Gals (189 Litres)

Fuel pressure

– normal range		:	35-550 mbar (Green arc)

Fuel consumption	:	Approx 6 Imp Gal/Hr @ 2300 RPM

Fuel drain position	:	Rear L.H.S of lower cowling + 1 drain for each wing tank and 2 drains for the main tank

Associated controls / indicators :

Fuel cock, mixture control, throttle, fuel contents gauge, fuel pressure gauge, carburettor heat control.

Low fuel pressure & level

warning light 	:	On annunciator panel

8.   IGNITION SYSTEM

Number / type of magnetos	-	Two x Bendix

Magneto switch settings		-	Off – R – L – Both

Impulse & spark retard device	- 			     (for engine start)		-	Left Magneto

Dead cut check performed at	-	1200 RPM

Magneto drop check

performed at			-	2000 RPM

Acceptable magneto

drop @ 2000 RPM			-	175 RPM

Acceptable difference

@ 2000 RPM			-	50 RPM

NB:		1. Magnetos are ground to earth when switched off.

2. Ignition key should only be removable with  magnetos  switched off.

9.   ELECTRICAL SYSTEM

Battery voltage/current :	12 Volts DC / 25 Ampere hours

Battery position	  :	Forward side of firewall, left hand side of engine

Alternator	  :		12 Volt / 60 Ampere

Associated controls / indicators:

Ammeter, circuit breakers, battery master switch, alternator switch, alternator warning light.

Electrically driven instruments & systems

Turn co-ordinator

Stall warning audio

Pitot head heater

Clock

Radios

Navigation aids

Internal and external lighting & strobe

Fuel booster pump and contents gauge

Starter motor and warning light

Engine instruments

Alternator warning light

Annunciator panel

10.   VACUUM SYSTEM

Vacuum pump				:  	Engine

driven

Vacuum pressure (normal range)	:  	Green arc

Minimum RPM for green arc		:  	1500 RPM

Vacuum driven gyro instruments	: 	AI & DI

Time to reach operating speed		:	2 minutes

Reliable time after vacuum failure	:	1 minute

Failure indication on AH and DI		:	Nil

11.   PITOT & STATIC PRESSURE SYSTEMS

Pitot tube location	:	Under leading edge port wing

Pitot heating		:	Electrical

Instrument supplied		:	Airspeed indicator

Static source locations	:	Each side of rear  fuselage

Instruments supplied	:	Airspeed indicator, Vertical speed indicator

Altimeter

12.   UNDERCARRIAGE

Type	:	Fixed, tricycle with shock absorbers,  pneumatic tyres and steerable nosewheel.

Tyre Pressures

- nosewheel	  	:	26 PSI

- mainwheels 		:	29 PSI

Oleo extensions 			:	Approx 3 inches

Nose wheel steering		:	Via rudder pedals

##### Wheel brakes		 :		Hydraulic disc brakes

##### Braking methods		  :	Toe brakes and parking brake

13.   CRITICAL SPEEDS

Vne	Velocity never exceed		166 	Kt

Vno	Velocity normal operations	140 	Kt

Va	Velocity manoeuvring		116	Kt

Vfe	Velocity flaps extended		  92	Kt

Lift-off speed (take-off flap)	  50	Kt

##### Lift-off speed (no flap)		  53	Kt

Best rate Climb speed

(take-off flap)		  81	Kt

(Flap up)				  92     Kt

Best angle Climb speed

(NB: Only when absolutely necessary, due to poor

engine cooling)

(take-off) flap)		  70	Kt

(Flap up)				  76     Kt

Max Turbulence speed (flap up)140     Kt

Powered approach

(Full flap)					  68	Kt

Best glide speed (range

approx 2nm per 1000 feet)	  81	Kt

Best endurance speed	 	  70	Kt

Threshold speed

(flaps extended)			  60	Kt

Threshold speed

(no flap)					  65	Kt

Vso	Velocity stall

(take off flap : 18o)			  53	Kt

Vso	Velocity stall

(full flap : 40o)				  51	Kt

Vs1	Velocity stall (no flap)		  57	Kt

Max cross-wind

for take-off & landing		  22	Kt

14.   CRITICAL WEIGHTS

Maximum all up weight

for take off and landing (Cat N)	        1100	Kg

Basic empty weight

(inc engine oil)					623	Kg

Maximum luggage

compartment weight				  60	Kg

Weight of full fuel

load (180 litres)				           130 	Kg

###### EMERGENCY DRILLS

1.	ENGINE FIRE IN FLIGHT

2.	ENGINE FIRE DURING START

3.	ELECTRICAL FIRE

4.	COCKPIT FIRE

5.	FUMES IN THE COCKPIT

6.	OIL PRESSURE FAILURE

7.	ENGINE MECHANICAL FAILURE

8.	ENGINE FAILURE - PROPELLER STOPPED

- ENGINE FAILURE - PROPELLER TURNING

- ENGINE FAILURE AFTER TAKE-OFF

11.	ROUGH RUNNING ENGINE

12.	ENGINE RESTART PROCEDURE

13.	FORCED LANDING CHECKS

14.	DITCHING

15.	ALTERNATOR FAILURE

16.	COMMUNICATIONS/RADIO FAILURE

1.	ENGINE FIRE IN FLIGHT

Fuel selector			OFF

Throttle	Fully open until engine stops – then closed

Mixture	ICO

Fuel Pump 	OFF

Alternator	OFF

Cabin heat/ventilation	OFF

Max Glide speed	81 Kts

Carry out Forced Landing Cx	See 13.

2.	ENGINE FIRE DURING START

Keep engine turning on starter

Fuel selector			OFF

Fuel pump				OFF

Throttle				Fully open

Mixture				ICO

Once fire is out turn off all electrical switches, vacate the aircraft  and place it u/s.

If fire continues

Magnetos				OFF

Battery 	 			OFF

Alternator 				OFF

Vacate the aircraft and fight fire with available equipment

3.	ELECTRICAL FIRE

Master Switch			OFF

Alternator				OFF

Circuit Breakers			Trip All

Cockpit Fire Drill			Action if necessary

Land as soon as possible

4.	COCKPIT FIRE

Fresh Air Vents			Open

Radio					Emergency call

Squawk 7700

Fire Extinguisher			As required*

For electrical fires

Cabin ventilation			Reduce

Alternator				OFF

Battery				OFF

Land ASAP if not extinguished.

*  Halon fire extinguishers are a toxic hazard

5.	FUMES IN THE COCKPIT

Cockpit Hot Air			OFF

Fresh Air Vents			Open

Engine Instruments		Check for sign of malfunction

If smell is electrical, carry out Electrical Fire checks

If smell is petrol, do not make electrical selection

Land as soon as possible

6.	LOW OIL PRESSURE

Throttle	Use minimum practicable power

Monitor Oil temperature

Assume engine failure is imminent

Land as soon as possible via precautionary forced landing pattern

If engine seizes carry out Engine Mechanical Failure  (7) and Forced Landing checks (13)

7.	ENGINE MECHANICAL FAILURE (Propeller stops suddenly perhaps with obvious signs of failure)

Adopt glide attitude and choose field for forced landing

Fuel				Throttle closed

Mixture ICO

Fuel cock OFF

Fuel pump OFF

Ignition			Magnetos OFF

Radio				Emergency call

Squawk 7700

Electrics			Master OFF

Alternator OFF

Harness			Secure

Alternator 			OFF

DO NOT ATTEMPT RESTART

On short final

Flaps 					Full

Battery				OFF

Canopy				Unlock

8.	ENGINE FAILURE (no signs of mechanical failure)

Select glide attitude and choose field for forced landing

If height sufficient (above 1500’ agl) attempt restart

Warning: If the engine failed with unusual mechanical noise do not attempt restart.

Radio 				Emergency call

Carburetor Heat			Change setting

Fuel selector			Select alternate tank					which contains fuel

Fuel pump 				ON

Mixture				Fully rich

Throttle				¼” open

Magnetos				Both

If engine does not start from windmilling operate starter

If engine has not started by 1500’ agl carry out Forced Landing checks

9.	ENGINE FAILURE-PROPELLER TURNING

If there is no Oil Pressure or an unusual mechanical noise carry out Engine Mechanical Failure drill, otherwise attempt restart procedure

10.	ENGINE FAILURE AFTER TAKE-OFF

Select glide attitude		78 kts

Radio					Emergency call

Select landing area within 30 of hdg

Flaps 					As rqd

#### If time permits

Mixture 				ICO

Fuel selector 			OFF

Fuel Pump 				OFF

Magnetos				OFF

Battery 				OFF

Canopy				Unlock

11.	ROUGH RUNNING ENGINE

Change carburetor heat setting

Fuel pump 			ON

-  Fuel Pressure 		Indicating

Fuel contents 		Sufficient

-  Fuel selector 		Change tanks

Mixture 			Fully rich

Ignition			Select best of L-R-BOTH

Electrics			Master ON

Alternator OFF

If engine fails to start, carry out Engine Mechanical Failure and Forced Landing checks.

12.	ENGINE RESTART PROCEDURE

Fuel				Fuel pump ON

Fuel cock ON

Throttle 1/4 open

Mixture full rich

Pressure checked

Contents checked

Ignition			Both

Electrics			Alternator OFF

Master ON

Either operate starter or carry out Air Start. Once the engine is running:

Throttle	Advance slowly-allow engine to warm

Alternator			ON

13.	FORCED LANDING CHECKS

Glide at:	(a)	65 kias - clean (still air glide range about	2nm/1000’)

(b)	60 kias - flap take-off/land

Select suitable landing area & plan engine out approach

If appropriate and time permitting, carry out:

(1)	Engine Restart Drill

(2)	Engine Mechanical Failure Drill

When committed to Forced Landing

Harness			Secure

Passengers			Brief

Fuel pump			OFF

Mixture			ICO

Throttle			Close

Magnetos			OFF

Fuel selector		OFF

Alternator			OFF

On short final

Flaps 					Full

Battery				OFF

Canopy				Unlock

14.	DITCHING

Warning: Ditching is best carried out whilst engine power is still available to control the rate of descent.

Flaps					LAND

Speed				55 kias

Rate of descent			300 fpm

Do not round out - continue descent into water

In strong wind, land into wind; otherwise land parallel to the swell

15.	ALTERNATOR FAILURE

Electrical Equipment	All off

Alternator			Off

Excitation CB		Set

Alternator CB		Set

Alternator			On

If alternator output restored, re-establish only essential electrical loads, land as soon as practicable

If alternator output not restored, use minimum electrical services and attain VMC. Battery duration approx 30 minutes.

16.	COMMUNICATIONS/RADIO FAILURE

Radio/Intercom

Switches			Check

Circuit Breakers		Check

Radio				Change frequency

Headset			Check connections

Change headsets

Radio				Switch Off/On

Transponder		Squawk 7600