Engine: Rotax 912UL 80HP

Propeller: Warp Drive 3 blade 68x21.5

PREFLIGHT - COCKPIT

Outbrief/Pax. brief	COMPLETED

Parking Brake 	SET

A/C Docs Folder 	PRESENT

Fuel Tap 	OPEN

Fuel Quantity 	CHECK

Ignition/Magnetos 	OFF

Master Key 	ON

Posi Lights 	ON

Strobe Lights 	ON

Lights 	CHECK EXTERNALLY

All Switches 	OFF

Master Key 	OUT

Instruments 	CHECK

Aileron Connections 	CHECK

Flaps 	FULL DOWN

Flight Controls 	FULL AND FREE

Rudder Cables 	CHECK

Central Flap 	CHECK FUEL CONTENTS

Straps 	CHECK

Loose Articles 	CHECK

PREFLIGHT - EXTERNAL

Chocks 	REMOVE

Pitot Cover 	REMOVE

FORWARD FUSELAGE

MASTER KEY OUT

ENSURE IGNITIONS OFF

Port Door 	CLOSED AND SECURE

Windscreen 	CLEAN

Port Cowling 	SECURE

Oil	GURGLE -  CHECK LEVEL

*FIRST FLIGHT OF DAY ONLY*

*Coolant Level 	CHECK RESERVOIR*

Oil Hatch 	SECURE

Propeller 	CHECK CLEAN INC. BACK

Spinner 	CHECK

Engine Intakes 	CLEAR

Nose Gear 	CHECK

Starboard Cowling 	SECURE

Starboard Door 	CHECK CLOSED

STARBOARD WING

Leading Edge 	CHECK CLEAN

Wing Under Surface 	CHECK

Wingtip 	CHECK

Wing Lift 	CARRY OUT

Upper Wing Surface 	CHECK

Flap 	SECURE

Wing Struts + Pin 	SECURE

REAR FUSELAGE

Stbd. landing gear 	CHECK INCLUDING  TYRE

Fuel Cap 	SECURE

Fuselage Fuel Drain 	CHECK FOR WATER

BRS Panel 	SECURE

Upper Fuselage 	CHECK

Fuselage Side 	CHECK

Empennage 	SECURE

Empennage Struts 	SECURE

Elevator Starboard 	FREE, CORRECT SENSE

Rudder 	CHECK CONNECTIONS

Tail Bumper 	CHECK

Elevator Port	FREE, CORRECT SENSE

Trim Tab 	SECURE

Empennage & Struts 	SECURE

Lower Fuselage 	CHECK

Fuselage Side 	CHECK

Inspection Panel 	REMOVE

Loose Objects 	CHECK

BRS System 	CHECK

Fuel Quantity 	CHECK

Fuel Filter and colour	CHECK

Control Linkages 	CHECK

Inspection Panel 	REPLACE

Port Landing Gear 	CHECK

PORT WING

Wing Struts + Pin 	SECURE

Flap 	SECURE

Skin Upper Surface 	CHECK

Aileron 	FREE, CORRECT SENSE

Wing Lift 	CARRY OUT

Wingtip 	CHECK

Wing Under Surface 	CHECK

Leading Edge 	CHECK CLEAN

Pitot Probe 	UNCOVER AND EXTEND

STARTING ENGINE

Parking Brake 	SET

Flaps 	UP

Flight Controls 	FULL AND FREE

Fuel Tap 	ON

Carb Heat 	ON (FLAP CLOSED)

Master Key 	ON

Meter	RECORD TIME

Fuel Pump 	ON 5s WHEN COLD

Ignition/Magnetos 	ON

Throttle 	CLOSED

Choke 	WHEN COLD

Area 	CLEAR AROUND

‘CLEAR PROP’

Starter 	ENGAGE

WHEN ENGINE RUNNING

Choke 	OFF

Oil Pressure 	RISING

Throttle 	2000 RPM

Comm/Intercom 	ON

Posi 	ON

Ignition/Magnetos 	LIVE/DEAD CUT

USB+EFIS 	ON

Transponder 	ON

AFTER 2 MINS

Throttle 	2500rpm

WHEN COOLANT >50°C

Radio Call 	REQUEST TAXI FOR….

Altimeter 	SET

TAXI

Brakes 	CHECK

Rudder 	FULL AND FREE

Compass 	SENSIBLE READING

BEFORE TAKEOFF

Position 	INTO WIND

Parking Brake 	SET

Throttle 	2000 RPM

Harness/Hatches 	SECURE

Loose Items 	STOWED

Flight Instruments 	SET

Position 	CLEAR AROUND

Throttle 	4000 RPM

Ignition/Magnetos	MAX 300 DROP, 125 DIFF.

Ts & Ps 	WITHIN LIMITS

Throttle 	MIN IDLE 1400 RPM

Throttle 	2000 RPM

Choke 	OFF

Strobe 	ON

Fuel Pump 	ON

Transponder 	ALT

Trim 	1 UNIT UP

Flaps 	ONE STAGE

BRS Safety Pin 	OUT

Departure Brief	COMPLETED

Carb Heat 	OFF

Flight Controls 	FULL AND FREE

Take off time 	RECORD

TAKEOFF ROLL

Power	5000-5800 RPM

Air Speed	INCREASING, REF. POINTS

Ts & Ps	WITHIN LIMITS

AFTER TAKEOFF

Flaps 	UP

Fuel Pump 	OFF

Ts & Ps	IN THE GREEN

ENROUTE - FREDA

Fuel

Radio

Engine

Direction

Altimeter

STALLING - HASELL

Height

Airframe

Security

Engine

Location

Lookout

BETWEEN MANOUEVRES - HELL

Height

Engine

Location

Lookout

AIRFIELD APPROACH CHECKS

Radio 	SET/VOL/REQUEST JOIN

Ts & Ps 	CHECK

Compass	HEADING SENSE

Altimeter 	SET QFE

DOWNWIND

Radio	DOWNWIND FOR….

Brakes	OFF

Carb Heat 	EXERCISE MIN 10s

Magnetos/Ignition	BOTH ON

Fuel 	SUFFICIENT, GO AROUND

Aux. pump	ON

Ts & Ps 	CHECK

Harnesses/Hatches	SECURE

FINAL

Flaps 	AS REQUIRED

Carb Heat 	ON

Radio 	CALL FINAL

Approach 	REASONABLE

Runway 	CLEAR

AFTER LANDING AND CLEAR OF RUNWAYS

Parking Brake 	SET

Throttle 	2000 RPM

Trim 	1 UNIT UP

Flaps 	UP

Fuel Pump 	OFF

Transponder 	SBY

BRS Pin 	REFIT

Landing Time	RECORD

Radio	REQUEST TAXI TO….

SHUTDOWN

Parking Brake 	SET

Throttle 	2000 RPM

Magnetos 	CHECK LIVE/DEAD CUT

USB+EFIS 	OFF

Strobe 	OFF

Posi 	OFF

Comm 	OFF

Throttle 	IDLE

Magnetos 	2 OFF, 1 OFF

Meter	RECORD

Master Key 	OUT

Fuel Tap 	CLOSED

VACATING AIRCRAFT

Chocks 	FITTED

Pitot Probe 	RETRACT/COVER

Master/Mags 	OFF

Parking Brake 	RELEASED

Flight Controls 	SECURE

Straps 	TIDY

Checklist 	KEEP IN AIRCRAFT

Tie Down 	AS REQUIRED

SUDDEN LOSS OF ENGINE POWER

Glide Speed 	58kts and TRIM

Initial Landing Site 	SELECT

Master Key 	ON

Magnetos 	ON

Fuel Tap 	ON

Choke 	OFF

Fuel Pump 	ON

Throttle 	1/4 OPEN

Starter 	ENGAGE

IF NO RESTART CONTINUE BELOW

EMERGENCY SHUTDOWN

Glide Speed 	58kts and TRIM

Landing Site 	SELECT

Mayday Call 	INITIATE

Throttle 	CLOSE

Magnetos 	OFF

Fuel Pump 	OFF

Fuel Tap 	OFF

Harness 	TIGHT, REASSURE PAX.

CONSIDER BRS ABOVE 500’

ENGINE FIRE

Fuel Tap 	OFF

Fuel Pump 	OFF

Throttle 	FULLY OPEN

Airspeed 	SIDE SLIP, PUT OUT FIRE

Radio	CALL MAYDAY

Land	ASAP

CONTROL LAMP ON

ELECTIRCAL MALFUNCTION

Electrical Load 	MINIMUM

LAND 	ASAP

Anticipate Electrical Failure

ELECTRICAL FAILURE

Electrical Load 	REDUCE

Circuit Breakers 	CHECK

Transponder 	BATT IND CHECK

IF NO POWER RESTORED

Airfield 	JOIN OVERHEAD

Light Signals 	OBSERVE

COMMUNICATION FAILURE

Headset 	PLUGS/CHANGE

Comms	ON

Intercom 	ON

Radio 	FREQ/VOL/SQUELCH

Circuit Breakers 	CHECK

IF NO COMMS ESTABLISHED

Transponder 	7600

Airfield 	JOIN OVERHEAD

Light Signals 	OBSERVE

| SPEEDS | ENGINE LIMITATIONS |
| --- | --- |
| Min sink 45kts flap 15° 450FPM |  |
| Lift off 50-55kts | Max cont. 5500 RPM |
| Vy 58kts flap 15° | Max 5800 RPM(5min) |
| Climb clean 62-66kts | FULL POWER |
| Cruise climb 70kts | POWER AS REQUIRED |
| Cruise@4500 RPM 83kts 12.7l/h |  |
| ATZ join 80kts | Oil pressure 2-5bar |

CIRCUIT & LAND WITH FLAP

|  | Oil Temp 50-140°C |
| --- | --- |
| Base Leg 63kts | Coolant Ideal 90-110°C |
| Approach 55kts | Coolant 50-120°C |
| Fence speed 52kts | (yellow triangle) |
| Landing 40-50kts | Fuel Pressure 0.15-0.4bar |

CIRCUIT AND LAND FLAPLESS

Approach 60kts

Landing    45-55kts

| SPEEDS | Persons, baggage and fuel |  |  |
| --- | --- | --- | --- |
| Vne 121kts | 379lb | 172kg | 15l |
| Vno 41-82kts | 374lb | 170kg | 19l |
| Vs1 41kts flaps up | 352lb | 160kg | 33l |
| Vs2 36kts first stage | 330lb | 150kg | 47l |
| Vso 32kts full flap | 308lb | 140kg | 61l |
| Vfe 63kts fully ext. | 302lb | 137kg | 65l |

Max. demonstrated X-Wind is steady 15kts

Max steady surface wind 22kts

Landing distance 205m from 15m fence

Take off distance 242m to clear 15m fence

AIRFRAME LIMITATIONS

Non-Aerobatic only

+4g / -2g

Max Bank 60 degrees

Day VFR

Non icing conditions

| MASS and BALANCE | CONVERSIONS |  |
| --- | --- | --- |
|  | Pound | Kg |
|  | 210 | 96 |
| G-CKYL weighs 288.2kg | 196 | 89 |
| MTOW 472.5kg with BRS | 182 | 83 |
| Aft of datum 366mm-560mm | 168 | 77 |
| Max cockpit load 172kg | 154 | 70 |
|  | 140 | 64 |