Engine: Rotax 912ULS 100 hp

Propeller: Neuform 3 blade 175cm 27⁰@310mm

Hex: 404D6F

PREFLIGHT - COCKPIT

Outbrief 	COMPLETED

Parking Brake 	SET

A/C Docs Folder 	PRESENT

Fuel Tap 	OPEN

Fuel Quantity 	CHECK

Ignition/Magnetos 	OFF

Master Key 	ON

Posi Lights 	ON

Strobe Lights 	ON

Lights 	CHECK EXTERNALLY

All Switches 	OFF

Master Key 	OFF/OUT

Instruments 	CHECK

Aileron Connections 	CHECK

Flaps 	FULL DOWN

Flight Controls 	FULL AND FREE

Rudder Cables 	CHECK

Central Flap 	CHECK

Straps 	CHECK

Loose Articles 	CHECK

PREFLIGHT - EXTERNAL

Chocks 	REMOVE

Pitot Cover 	REMOVE

FORWARD FUSELAGE

MASTER KEY OFF/OUT

ENSURE IGNITIONS OFF

Port Door 	CLOSED AND SECURE

Windscreen 	CLEAN

Port Cowling 	SECURE

Oil Level 	BURP CHECK LEVEL

*FIRST FLIGHT OF DAY ONLY*

*Coolant Level 	CHECK RESERVOIR*

Oil Hatch 	SECURE

Propeller 	CHECK

Spinner 	CHECK

Engine Intakes 	CLEAR

Nose Gear 	CHECK

Stbd Cowling 	SECURE

Stbd Door 	CHECK/SECURE

STARBOARD WING

Leading Edge 	CHECK

Wing Under Surface 	CHECK

Wingtip 	CHECK

Wing Lift 	CARRY OUT

Upper Wing Surface 	CHECK

Flap 	SECURE

Wing Struts + Pin 	SECURE

REAR FUSELAGE

Stbd Landing Gear 	CHECK

Fuel Cap 	SECURE

Fuselage Fuel Drain 	SAMPLE

Fuselage Panel 	SECURE

Upper Fuselage 	CHECK

Fuselage Side 	CHECK

Empennage 	SECURE

Empennage Struts 	SECURE

Elevator Port 	FREE, CORRECT SENSE

Rudder 	SECURE

Tail Bumper 	CHECK

Elevator Starboard	FREE, CORRECT SENSE

Trim Tab 	SECURE

Empennage & Struts 	SECURE

Lower Fuselage 	CHECK

Fuselage Side 	CHECK

Inspection Panel 	REMOVE

Loose Objects 	CHECK

Fuel Quantity 	CHECK

Fuel Filter 	CHECK

Control Linkages 	CHECK

Inspection Panel 	REPLACE

Port Landing Gear 	CHECK

PORT WING

Wing Struts + Pin 	SECURE

Flap 	SECURE

Skin Upper Surface 	CHECK

Aileron 	FREE, CORRECT SENSE

Wing Lift 	CARRY OUT

Wingtip 	CHECK

Wing Under Surface 	CHECK

Leading Edge 	CHECK

Pitot Probe 	CHECK EXTENDED

STARTING ENGINE

Parking Brake 	SET

Flaps 	UP

Flight Controls 	FULL AND FREE

Fuel Tap 	ON

Master Key 	ON

Meter	RECORD

Fuel Pump 	ON 5secs WHEN COLD

Ignition/Magnetos 	ON

Throttle 	CLOSED

Choke 	WHEN COLD

Area 	CLEAR AROUND

‘CLEAR PROP’

Starter 	ENGAGE

WHEN ENGINE RUNNING

Choke 	OFF

Oil Pressure 	RISING

Throttle 	2000 RPM

Comm/Intercom 	ON

Posi 	ON

Ignition/Magnetos 	LIVE/DEAD CUT

USB+EFIS 	ON

Transponder 	ON

AFTER 2 MINS

Throttle 	2500rpm

WHEN COOLANT >50°C

Radio Call 	REQUEST TAXI FOR….

Altimeter 	SET

TAXI

Brakes 	CHECK

Rudder 	FULL AND FREE

Compass 	CORRECT SENSE

BEFORE TAKEOFF

Position 	INTO WIND

Parking Brake 	SET

Throttle 	2000 RPM

Harness/Hatches 	SECURE

Loose Items 	STOWED

Flight Instruments 	SET

Position 	CLEAR AROUND

Throttle 	4000 RPM

Ignition/Magnetos	MAX 300 DROP, 125 DIFF.

Ts & Ps 	WITHIN LIMITS

Throttle 	MIN IDLE 1400 RPM

Throttle 	2000 RPM

Choke 	OFF

Strobe 	ON

Fuel Pump 	ON

Transponder 	ALT

Trim 	1 UNIT UP

Flaps 	ONE STAGE

Departure Brief	COMPLETED

Carb Heat 	OFF

Flight Controls 	FULL AND FREE

Time 	RECORD

TAKEOFF ROLL

Power	5000-5800 RPM

Air Speed	INCREASING, REF. POINTS

Ts & Ps	WITHIN LIMITS

AFTER TAKEOFF

Flaps 	UP

Fuel Pump 	OFF

Ts & Ps	IN THE GREEN

ENROUTE - FREDA

Fuel

Radio

Engine

Direction

Altimeter

STALLING - HASELL

Height

Airframe

Security

Engine

Location

Lookout

BETWEEN MANOUEVRES - HELL

Height

Engine

Location

Lookout

AIRFIELD APPROACH CHECKS

Radio 	SET/VOL/REQUEST JOIN

Ts & Ps 	CHECK

Compass	HEADING SENSE

Altimeter 	SET QFE

DOWNWIND

Radio	DOWNWIND FOR….

Brakes	OFF

Magnetos/Ignition	BOTH ON

Fuel 	SUFFICIENT, GO AROUND

Aux. pump	ON

Ts & Ps 	CHECK

Hatches/Harnesses	SECURE

FINAL

Flaps 	AS REQUIRED

Radio 	CALL FINAL

Approach 	REASONABLE

Runway 	CLEAR

AFTER LANDING AND CLEAR OF RUNWAYS

Parking Brake 	SET

Throttle 	2000 RPM

Trim 	1 UNIT UP

Flaps 	UP

Fuel Pump 	OFF

Transponder 	SBY

Landing Time	RECORD

Radio	REQUEST TAXI FOR….

SHUTDOWN

Parking Brake 	SET

Throttle 	2000 RPM

Magnetos 	CHECK LIVE/DEAD CUT

USB+EFIS 	OFF

Strobe 	OFF

Posi 	OFF

Comm 	OFF

Throttle 	IDLE

Magnetos 	2 OFF, 1 OFF

Meter	RECORD

Master Key 	OFF/OUT

Fuel Tap 	CLOSED

VACATING AIRCRAFT

Chocks 	FITTED

Pitot Probe 	RETRACT/COVER

Master/Mags 	OFF

Parking Brake 	RELEASED

Flight Controls 	SECURE

Straps 	TIDY

Checklist 	PUT IN AIRCRAFT

Tie Down 	AS REQUIRED

SUDDEN LOSS OF ENGINE POWER

Glide Speed 	58kts and TRIM

Initial Landing Site 	SELECT

Master Key 	ON

Magnetos 	ON

Fuel Tap 	ON

Choke 	OFF

Fuel Pump 	ON

Throttle 	1/4 OPEN

Starter 	ENGAGE

IF NO RESTART CONTINUE BELOW

EMERGENCY SHUTDOWN

Glide Speed 	58kts and TRIM

Landing Site 	SELECT

Mayday Call 	INITIATE

Throttle 	CLOSE

Magnetos 	OFF

Fuel Pump 	OFF

Fuel Tap 	OFF

Harness 	TIGHT, REASSURE PAX.

ENGINE FIRE

Fuel Tap 	OFF

Fuel Pump 	OFF

Throttle 	FULLY OPEN

Airspeed 	SIDE SLIP, PUT OUT FIRE

Radio	CALL MAYDAY

Land	ASAP

CONTROL LAMP ON

ELECTIRCAL MALFUNCTION

Electrical Load 	MINIMUM

LAND 	ASAP

Anticipate Electrical Failure

ELECTRICAL FAILURE

Electrical Load 	REDUCE

Circuit Breakers 	CHECK

Transponder 	BATT IND CHECK

IF NO POWER RESTORED

Airfield 	JOIN OVERHEAD

Light Signals 	OBSERVE

COMMUNICATION FAILURE

Headset 	PLUGS/CHANGE

Comms	ON

Intercom 	ON

Radio 	FREQ/VOL/SQUELCH

Circuit Breakers 	CHECK

IF NO COMMS ESTABLISHED

Transponder 	7600

Airfield 	JOIN OVERHEAD

Light Signals 	OBSERVE

SPEEDS 	ENGINE LIMITATIONS

Glide speed 58kts

Min sink 45kts 450FPM

Lift off 50-55kts

Vy 58kts flap 15°	Max 5800 RPM(5min)

Climb clean 62-66kts	FULL POWER

Cruise climb 70kts	POWER AS REQUIRED

Cruise @4500 RPM 83kts, 12.7 l/h

ATZ join 80kts	Oil pressure 2-5bar

CIRCUIT & LAND WITH FLAP

Oil Temp 50-140°C

Base Leg 63kts	Coolant Ideal 90-110°C

Approach 55kts	Coolant 50-120°C

Fence speed 52kts	(yellow triangle)

Landing    40-50kts	Fuel Pressure 0.15-0.4bar

CIRCUIT AND LAND FLAPLESS

Approach 60kts

Landing    45-55kts

SPEEDS	Payload – Fuel limitation

Vne 121kts	186kg - 	25 litres

Vno 41-82kts	179kg - 	35 litres

Vs1 41kts flaps up	168kg - 	50 litres

Vs2 36kts first stage	158kg - 	65 litres

Vso 32kts full flap

Vfe 63kts fully extended at top of white arc.

Max. demonstrated X-Wind is steady 15kts

Max steady surface wind 22kts

Landing distance 205 - 234m from 15m fence

Take off distance 205m to clear 15m fence

AIRFRAME LIMITATIONS

Non-Aerobatic only

+4g / -2g

Max Bank 60 degrees

Day VFR

Non icing conditions

MASS and BALANCE		    CONVERSIONS

Pound	Kg

210	96

G-OSTL weighs *268.2kg 	196	89

MTOW 472.5kg	182	83

Aft of datum 366mm-560mm	168	77

Max cockpit load 172kg	154	70

140	64

*245.7kg where BRS has been removed