## PREFLIGHT INSPECTION

#### Aircraft documents	VALID AND COMPLETE.

#### Control wheel lock	REMOVE.

#### Parking brake	ON.

#### Master switch	ON.

#### Magnetos	OFF.

#### Fuel	SELECT BOTH.

#### Fuel quantity gauges	NOTE READINGS.

#### Lights	OFF*.

#### Flaps	DOWN.

#### Overhead switches	OFF*.

#### Master switch	OFF*.

#### Fire extinguisher	IN PLACE.

#### First aid kit	IN PLACE.		* Except night flying.

## WALK AROUND CHECKS – PORT WING.

#### Flap			CHECK SECURITY, FLAP TRACKS, OPERATING LINKAGES FREE, NUTS SECURE AND DRAIN HOLES CLEAR.

#### Fuel tank sump strainer	SAMPLE/DRAIN FUEL BEFORE FIRST FLIGHT/AFTER REFUELLING TO CHECK FOR WATER, SEDIMENT AND CORRECT FUEL GRADE.

#### Aileron		CHECK FREEDOM OF MOVEMENT AND SECURITY.

#### Nav. and strobe lights	SECURE AND CORRECT COLOUR.

#### Leading edge		CHECK CONDITION.

#### Pitot tube and fuel vent	CLEAR

#### Lower wing surface	CHECK CONDITION AND CLEANLINESS.

#### Upper wing surface	CHECK CONDITION AND FREE FROM ICE/SNOW/FROST.

#### Fuel tank		CHECK CONTENTS, SECURE CAP.

#### Tyre			CHECK CONDITION, INFLATION AND CREEP.

#### Brake pads, hydraulic lines	CHECK.

## NOSE GEAR

#### Tyre			CHECK CONDITION, INFLATION AND CREEP.

#### Oleo			CHECK EXTENSION 3.25 INCHES.

#### Torque link		SECURITY OF LINKAGES.

## FRONT FUSELAGE AND ENGINE

#### Windscreen		CHECK CONDITION AND CLEANLINESS.

#### Static vent		CLEAR.

#### Propeller and spinner	CHECK FOR NICKS AND SECURITY. TREAT AS LIVE.

#### Carburretor air filter	CLEAR.

#### Cooling air intakes	CLEAR

#### Alternator belt		CHECK TENSION.

#### Engine		OIL LEVEL BETWEEN 4 AND 6 QUARTS, CHECK FILLER CAP FINGER TIGHT.

#### Fuel strainer		SAMPLE/DRAIN FOR FOUR SECONDS TO CLEAR WATER AND SEDIMENT BEFORE FIRST FLIGHT/AFTER REFUELLING. CHECK  CORRECT FUEL COLOUR.

## STARBOARD WING.

#### Fuel tank		CHECK CONTENTS, SECURE CAP.

#### Upper wing surface	CHECK CONDITION, FREE FROM ICE/SNOW/FROST.

#### Tyre			CHECK CONDITION, INFLATION AND CREEP.

#### Brake pads, hydraulic lines	CHECK.

#### Leading edge		CHECK CONDITION.

#### Lower wing surface	CHECK CONDITION AND CLEANLINESS.

#### Nav. and strobe lights	SECURE.

#### Aileron		CHECK CONDITION, SECURITY AND FREEDOM OF MOVEMENT.

#### Flap			CHECK SECURITY, FLAP TRACKS, OPERATING LINKAGES FREE, NUTS SECURE AND DRAIN HOLES CLEAR.

#### Fuel tank sump strainer	SAMPLE/DRAIN.

## STARBOARD FUSELAGE

#### Fuselage skin		CHECK CONDITION.

#### Radio aerials		SECURE.

#### Fin and tailplane	SECURE.

#### Elevator and trim tab	CHECK CONDITION AND FREEDOM OF MOVEMENT.

#### Rudder		CHECK FIXINGS, CABLE AND HINGES SECURE.

#### Lights		CHECK SECURITY.

## PORT FUSELAGE

#### Fuselage skin		CHECK CONDITION.

## INTERNAL.

#### Seat			ADJUSTED, HARNESSES SECURE.

#### Parking brake		ON.

#### Trim and controls	FULL AND FREE MOVEMENT IN THE CORRECT SENSE.

#### Master/Alternator switches	ON, ALTERNATOR WARNING LIGHT ON.

#### Flap			SELECT UP, CHECK VISUALLY.

#### Carb. Heat		EXERCISE SET COLD.

#### Fuel			SELECT BOTH.

#### Mixture		CHECK MOVEMENT, SET RICH.

#### Throttle		CHECK MOVEMENT, FRICTION NUT LOOSE, CLOSE.

#### Carb. Heat		EXERCISE SET COLD.

#### Fin light		ON.

#### Landing light		OFF.

#### Strobes		OFF.

#### Pitot heat		OFF.

#### Radio and Navaids	OFF.

#### Nav. and panel lights	OFF.

#### Radio lights		OFF.

#### Circuit breakers		ALL IN.

#### Instruments and gauges	CLOCK ACCURATE, ASI VERTICAL, QFE READING WITHIN +50' and -70'

#### Turn coordinator	FUNCTIONING.

#### DI 			TURN AND SET

#### VSI			Zero  200’ (NOTE ERROR).

#### Compass		SENSIBLE HEADING, NO DISCOLORATION AND NO BUBBLES.

#### OAT			SENSIBLE READING.

#### Hobbs meter		NOTE READING.

## STARTING

#### Primer		THREE AND SIX SHOTS WHEN REQUIRED, THEN LOCK.

#### Throttle		SET ¼” OPEN.

#### Area			LOOKOUT, SHOUT ‘CLEAR PROP’.

#### Starter		OPERATE (RELEASE AS SOON AS ENGINE FIRES).

#### Starter warning light	OUT.

#### Low voltage warning light	OUT.

#### Power		SET 1200 RPM.

## AFTER STARTING CHECKS.

#### Oil pressure		RISING WITHIN 30 SECONDS.

#### Fuel pressure		IN THE GREEN AND ABOVE PREVIOUS READING.

#### Ammeter		NORMAL READING, MAX. 30 AMPS.

#### Suction		INDICATING.

#### Magnetos		CHECK FOR DEAD CUT.

#### Radio		INTERCOM AND PHONES ON.

#### Navaids		CHECK, IDENT AND SET IF REQUIRED FOR DETAIL.

#### Pitot Heat		ON.

#### DI			SYNCHRONISE WITH COMPASS.

#### AI			SET.

#### Pitot Heat		OFF.

#### Landing light		ON AT NIGHT.

#### Clearance		REQUEST TAXI FOR LOCAL VFR FLIGHT/CIRCUITS

## TAXIING CHECKS.

#### Brakes		CHECK OPERATION GENTLY, CONTINUE MOVING.

#### Rudder and nose gear steering FULL AND FREE MOVEMENT.

#### Flight instruments	TC INDICATES TURN RIGHT, BALL OUT TO THE LEFT, DI INCREASES, COMPASS ALSO. TC INDICATES TURN LEFT, BALL OUT TO THE RIGHT, DI DECREASES, COMPASS ALSO. AI STEADY, ALTIMETER STEADY.

## POWER CHECKS.

#### Position		INTO WIND AND NOSEWHEEL STRAIGHT.

#### Throttle		SET 1700 RPM, CHECK BRAKES HOLDING.

#### Carb. Heat		OPERATE 10 SECONDS, CHECK MAX. DROP 75 RPM.

#### Magnetos		CHECK LEFT AND RIGHT DROP DOES NOT EXCEED 125 RPM. MAX. DIFFERENCE 50 RPM.

#### Oil T’s & P’s		STEADY.

#### Alternator load		AMMETER READING NORMAL (MAX. 30 AMPS.)

#### Suction		4.6 – 5.4 INCHES OF MERCURY.

#### Throttle		CLOSE, CHECK IDLE BETWEEN 500 AND 700 RPM, RESET 1200 RPM. DO NOT IDLE BELOW 1200 RPM.

## ALTIMETER SETTINGS

#### Circuits		SET QFE, CHECK ±50'.

#### Local		SET AERODROME QNH, CHECK ±50'.

#### Cruise		SET ZONE/TMA/REGIONAL QNH BELOW 3000’. ABOVE TRANSITION LEVEL SET QNE.  FLY QUADRANTAL RULES.

#### Circuit join		SET QFE WITH AIRFIELD IN SIGHT.

#### Missed Approach	SET AERODROME QNH.

## PRE TAKE OFF CHECKS.

#### Trimmer		SET TAKE-OFF.

#### Throttle		FRICTION NUT FINGER TIGHT.

#### Mixture		RICH(below 3000’), CARB. HEAT COLD.

#### Magnetos		BOTH.

#### Fuel selector valve	ON BOTH.

#### Flaps		10 FOR SOFT FIELD.

#### Gyros		AI ERECT, DI SYNCHRONISED.

#### Hatches		CLOSED AND LOCKED.

#### Harnesses		SECURE.

#### Controls		FULL AND FREE MOVEMENT

## DURING TAKE-OFF CHECKS.

#### Heels		WELL BACK TO AVOID TOE BRAKES.

#### Power		FULL THROTTLE, 2280 TO 2400 RPM, T’S & P’S GREEN.

#### ASI			INCREASING.

## AFTER TAKE-OFF CHECKS.

#### Brakes		APPLY MOMENTARILY.

#### Lookout		SELECT POINT LEFT OF NOSE.

#### Flaps		SLOWLY RETRACT AT 200’ (ACCELERATE TO 78 KIAS).

#### Landing light		OFF.

#### Radio		SELECT FREQUENCY REQUIRED.

#### Altimeter		SET AS REQUIRED.

## TOP OF CLIMB.

#### Fuel			CHECK CONTENTS, FUEL SELECTOR BOTH.

#### Engine		T’s & P’S IN THE GREEN, CARB. HEAT FOR 10 SECONDS.

#### Mixture		LEAN AS REQUIRED.

#### DI			RESYNCHRONISE.

#### Orientation		SELECT REFERENCE POINT.

## EN-ROUTE / AIRFIELD APPROACH CHECKS.

#### Fuel			CHECK CONTENTS, FUEL SELECTOR BOTH, MIXTURE SET.

#### Radios		TUNED, CORRECT FREQUENCY AND VOLUME.

#### Engine		T’s & P’S IN THE GREEN, CARB. HEAT ON FOR 10 SECONDS BEFORE CLOSING THROTTLE.

#### DI			RESYNCHRONISE.

#### Altimeter		SET QFE.

## PRE-STALLING / INCIPIENT SPINNING CHECKS

#### Height		SUFFICIENT TO RECOVER BY 2000’ agl DUAL.

#### Airframe		FLAPS AS REQUIRED.

#### Security		NO LOOSE ARTICLES, HATCHES CLOSED, HARNESSES SECURE.

#### Engine		T’s & P’S IN THE GREEN, MIXTURE RICH, SELECTED, CARB. HEAT FOR 10 SECONDS.

#### Location		CLEAR OF ACTIVE AIRFIELDS, BUILT-UP AREAS, CONTROLLED AIRSPACE.

#### Lookout		CLEAR OF OTHER AIRCRAFT AND CLOUD BOTH HORIZONTALLY AND VERTICALLY, CLEARING TURNS LEFT AND RIGHT THROUGH 90.

## AFTER STALLING CHECKS.

#### Fuel			CHECK CONTENTS, FUEL SELECTOR BOTH.

#### Radio		TUNED AND CORRECT.

#### Engine		T’s & P’s IN THE GREEN.

#### DI			RESYNCHRONISE.

#### Altimeter		SET QNH.

## PRE - LANDING CHECKS.

#### Brakes		OFF.

#### Undercarriage		DOWN AND LOCKED.

#### Mixture		RICH, CARB. HEAT ON

#### Fuel			ON, SUFFICIENT FOR GO AROUND, PRIMER IN AND LOCKED.

#### Instruments		T’s & P’s IN THE GREEN, SUCTION INDICATING, AMMETER READING NORMAL, DI SYNCHRONISED.

#### Carb. Heat		COLD

#### Hatches		CLOSED AND LATCHED.

#### Harnesses		SECURE.

## GO AROUND.

#### Throttle		FULL POWER.

#### Carb. Heat		COLD.

#### Flaps		20 IMMEDIATELY.

#### Airspeed		55 KIAS.

#### Nose			TO CLIMB ATTITUDE.

#### Flaps		10 UNTIL OBSTACLE CLEARED.

## AFTER LANDING CHECKS.

#### Position		WELL CLEAR OF ACTIVE RUNWAY AND STOP.

#### Flaps		UP.

#### Throttle friction nut	LOOSE.

#### Trim			TAKEOFF.

#### Carb. Heat		COLD.

#### Landing light		OFF (EXCEPT NIGHTS).

#### Electrics		NON ESSENTIAL LOADS OFF e.g. PITOT HEAT.

## SHUT DOWN CHECKS.

#### Position		INTO WIND, NOSE WHEEL STRAIGHT.

#### Toe brakes		ON.

#### Throttle		SET 1200 RPM.

#### Radios		OFF.

#### Magnetos		CHECK DEAD CUT AND LIVE MAG.

#### Mixture		IDLE CUT OFF.

#### Throttle		AS REQUIRED.

#### Magnetos		OFF WHEN ENGINE STOPS, KEY OUT.

#### Hobbs meter time	RECORD READING.

#### Master		OFF.

#### Fuel			OFF.

#### Seats		FULLY BACK.

#### Control lock		INSTALL.

#### Cockpit		CLEAN AND HARNESSES TIDY FOR NEXT PILOT.

#### Tie downs		IN PLACE.

## MAXIMUM LANDING WEIGHT – 2 300 lbs.

#### 75% cruise speed	116 KTAS @ 2000’.

#### 65% cruise speed	110 KTAS @ 2000’.

#### Vr			55 KIAS.

#### Vx			59 KIAS.

#### Vy			78 KIAS.

#### Vfe			85 KIAS.

#### Va			128 KIAS.

#### Vne			160 KIAS.

#### Cruise climb.		70 - 85 KIAS.

#### Powered approach	60 - 70 KIAS.

#### Powered (fence speed)	60 KIAS.

#### Glide			65 KIAS.

#### Flapless		60 - 70 KIAS.

#### Short over threshold	60 KIAS(until flare).

#### Demonstrated X/wind limit.	15 Kts.

#### Take-Off Run.		805’ x 1.3 x 1.43 = 1497’ (short wet grass, public transport)

#### Take-Off Distance.	1440’ x 1.3 x 1.43 = 2677’ (short wet grass, public transport)

#### Landing roll.		520’ x 1.3 x 1.43 = 967’ (after short field approach)

#### Landing Distance.	1250’ x 1.3 x 1.43 = 2324’ (50’ obstacle, short field approach)

#### Tyre pressures main/nose	26 psi / 29 psi

#### Nose gear shock strut	Fill with air to 45 psi

## ENGINE STOPS IN FLIGHT

#### Airspeed		65 KIAS.

#### Fuel			ON BOTH.

#### Mixture		RICH.

#### Throttle		SET ¼” OPEN.

#### Starter		OPERATE

## ENGINE FAILURE DURING CRUISE

#### Fuel			ON BOTH.

#### Carb. Heat		ON

## ELECTRICAL FAILURE

#### ALT annunciator on:

#### Ammeter		INDICATING ZERO.

#### Load			LANDING LIGHT OR OTHER LOAD ON, CHECK AMMETER

#### If reading does not increase:

#### Load			REDUCE.

#### ALT circuit breaker	CHECK AND RESET.

#### Next step, reset overvoltage relay by:

#### ALT switch		OFF FOR ONE SECOND THEN ON.

#### If power cannot be restored:

#### ALT switch		OFF

#### Load			MINIMUM, BATTERY POWER ONLY, LAND AS SOON AS PRACTICAL.

## ELECTRICAL OVERLOAD (20 AMPS ABOVE KNOWN ELECTRICAL LOAD)

#### ALT switch		ON

#### BATT switch		OFF (LOW BATTERY, BATTERY FAULT OR ABNORMAL LOAD

#### If alternator loads are reduced:

#### Load			MINIMUM, BATTERY POWER ONLY AVAILABLE, LAND AS SOON AS PRACTICAL.

#### If alternator loads are not reduced:

#### ALT switch		OFF

#### BATT switch		AS REQUIRED, LAND AS SOON AS POSSIBLE. ANTICIPATE COMPLETE ELECTRICAL FAILURE