# C-421B CHECKLIST

## Preflight inspection.

#### Aircraft documents – VALID AND COMPLETE.

#### Control locks – REMOVE AND STOW.

#### Parking brake – SET.

#### Static source selector – NORMAL.

#### Switches – ALL OFF (MAGNETOS, FUEL PUMPS, LIGHTS, NAV/COM RADIOS, TRANSPONDER, ADF AND AUTOPILOT).

#### Circuit breakers – IN.

#### Landing gear lights – THREE GREENS.

#### Battery switch – ON.

#### Beacon – ON.

#### Fuel gauges – CHECK QUANTITIES AND OPERATION.

#### Master switch – OFF.

#### Fire extinguisher – SECURE.

#### First aid kit – IN PLACE.

#### Flaps – DOWN.

#### Fuel – SELECT MAIN TANKS (FEEL FOR DETENTS).

#### Trim tabs - NEUTRAL.

#### Windshield and windows – CHECK FOR CRACKS.

#### Oxygen – CHECK, QUANTITY, MASKS AND HOSES - OFF

## Left wing.

#### Battery compartment cover – SECURE.

#### Wing locker baggage door – SECURE.

#### Flap – CHECK SECURITY AND ATTACHMENT.

#### Bottom outboard wing – CHECK FOR FUEL STAINS.

#### Control surface lock – REMOVE.

#### Upper wing surface – CHECK CONDITION AND FREE FROM ICE, SNOW AND FROST.

#### Aileron and tab – CHECK CONDITION, FREEDOM OF MOVEMENT AND TAB POSITION.

#### Main tank fuel sump - DRAIN.

#### Fuel vent and sniffle valve – CLEAR.

#### Main tank fuel quantity – CHECK, SECURE CAP.

#### Tip tank transfer pump – LISTEN FOR OPERATION.

#### Auxiliary tank fuel quantity – CHECK, SECURE CAP.

#### Stall warning vane – CHECK FREEDOM OF MOVEMENT AND AUDIBLE WARNING.

#### Wing tie down – REMOVE.

#### Deice boot – CHECK CONDITION AND SECURITY.

#### Auxiliary tank and wing locker transfer line fuel sump – DRAIN.

#### Engine compartment general condition – CHECK FOR FUEL, OIL AND EXHAUST LEAKS OR STAINS.

#### Engine – OIL LEVEL BETWEEN 9 AND 10 QUARTS FOR FLIGHTS OF LESS THAN THREE HOURS.

#### Propeller and spinner – EXAMINE FOR NICKS, SECURITY AND OIL LEAKS.

#### Engine opening – CLEAR.

#### Intake air opening – CLEAR.

#### Tyre  - CHECK CONDITION, INFLATION AND CREEP.

#### Oleo piston – CHECK EXTENSION 3.0”.

#### Wheel well doors – CHECK CONDITION

#### Fuel strainer – DRAIN.

#### Crossfeed line – DRAIN.

#### Heat exchanger opening – CLEAR.

## Nose.

#### Baggage door – SECURE.

#### Tyre  - CHECK CONDITION, INFLATION AND CREEP.

#### Oleo piston – CHECK EXTENSION 3.0”.

#### Torque link – SECURITY OF LINKAGES.

#### Pitot tube – CLEAR.

#### Lower fuselage, nose and center section – CHECK FOR FUEL STAINS.

#### Ram air inlet – CLEAR.

#### Heater inlet and outlet – CLEAR.

#### Windscreen – CHECK CONDITION AND CLEANLINESS.

#### Baggage door – SECURE.

## Right wing.

#### Heat exchanger opening – CLEAR.

#### Crossfeed line – DRAIN.

#### Fuel strainer – DRAIN.

#### Wheel well doors – CHECK CONDITION

#### Oleo piston – CHECK EXTENSION 3.0”.

#### Tyre  - CHECK CONDITION, INFLATION AND CREEP.

#### Intake air opening – CLEAR.

#### Engine – OIL LEVEL BETWEEN 9 AND 10 QUARTS FOR FLIGHTS OF LESS THAN THREE HOURS.

#### Engine compartment general condition – CHECK FOR FUEL, OIL AND EXHAUST LEAKS OR STAINS.

#### Propeller and spinner – EXAMINE FOR NICKS, SECURITY AND OIL LEAKS.

#### Engine opening – CLEAR.

#### Auxiliary tank and wing locker transfer line fuel sump – DRAIN.

#### Deice boot – CHECK CONDITION AND SECURITY.

#### Auxiliary tank fuel quantity – CHECK, SECURE CAP.

#### Tip tank transfer pump – LISTEN FOR OPERATION.

#### Main tank fuel quantity – CHECK, SECURE CAP.

#### Fuel vent and sniffle valve – CLEAR.

#### Main tank fuel sump - DRAIN.

#### Aileron – CHECK CONDITION AND FREEDOM OF MOVEMENT.

#### Bottom outboard wing – CHECK FOR FUEL STAINS.

#### Flap – CHECK SECURITY AND ATTACHMENT.

#### Wing locker baggage door – SECURE.

## Fuselage.

#### Static port – CLEAR.

#### Deice boots – CHECK.

#### Control surface locks – REMOVE RUDDER LOCK.

#### Elevator and tab – CHECK CONDITION, FREEDOM OF MOVEMENT AND TAB POSITION.

#### Rudder and tab – CHECK CONDITION, FREEDOM OF MOVEMENT AND TAB POSITION.

#### Static port – CLEAR.

#### Cabin door and seal – CHECK SECURITY AND CONDITION.

#### Battery switch – OFF.

## Before starting engines.

#### Preflight inspection – COMPLETE.

#### Cabin door – LATCHED;  Safety pin – INSTALLED.

#### Control lock – REMOVED.

#### Seats – ADJUSTED, HARNESSES SECURE.

#### Flying controls – FULL AND FREE MOVEMENT IN THE CORRECT SENSE.

#### Brakes – TEST AND SET.

#### Landing gear switch – DOWN.

#### Emergency alternator field switch – OFF.

#### Emergency avionics bus switch – OFF.

#### Circuit breakers – IN.

#### All switches - OFF

#### Avionics bus switch – OFF.

#### Auxiliary fuel pump switches – OFF.

#### Battery and alternators – ON.

#### Annunciator light panel – PRESS TO TEST.

#### Cabin door ‘Not locked light’ – OFF.

#### Altimeter and clock – SET.

#### Fuel quantity – CHECK.

#### Cabin air controls – AS REQUIRED.

#### Throttles – OPEN ONE INCH.

#### Propellers – FULL FORWARD (Fine pitch).

#### Mixtures – FULL RICH.

#### Master light dimming switch – AS REQUIRED.

#### Landing gear position indicator lights – CHECK THREE GREENS.

#### Pressurization air controls – PUSH IN AND LOCK.

#### Cabin pressurization switch - PRESSURIZE

#### Cabin vent control – PULL OUT (FOR ADDITIONAL GROUND VENTILATION)

#### Cabin rate – ARROW UP.

#### Cabin Altitude – SET 500 FEET ABOVE FIELD PRESSURE ALTITUDE.

#### Alternate air controls – IN.

#### Anti-collision lights – ON.

#### Hobbs meter reading – RECORD.

## Starting engines.

#### Propellers – CLEAR.

#### Magneto switches – ALL ON.

#### Starter button – PRESS; Primer switch – left engine - LEFT.

#### Auxiliary fuel pump - LOW

#### Starter warning light – OUT.

#### Throttle – 750 to 900 RPM.

#### Oil pressure – 10 PSI AND RISING WITHIN 30 SECONDS.

#### Right engine – START (repeat steps 1 through 7).

#### Alternators – CHECK.

#### Flaps – UP.

#### Avionics bus switch - ON

#### Radios - SET.

## Starting hot engine.

#### Primer switch – OFF.

#### Throttle – SET 1/2” OPEN.

#### Mixture – IDLE CUT OFF.

#### Area – LOOKOUT, SHOUT ‘CLEAR PROP’.

#### Starter – PRESS.

#### Mixture – FULL RICH WHEN ENGINE FIRES, RELEASE STARTER.

#### Throttle – TO IDLE.

#### Auxiliary fuel pumps – LOW.

## Before taxiing.

#### Beacon – CHECK ON.

#### Strobe lights – ON.

#### Radios –ON (ADF to REC, COMM 1 and COMM 2 ON, NAV 1 and NAV 2 to IDENT, Transponder to STANDBY and AUDIO SWITCHES TO PHONES.

#### ADF – REC to tune and identify (SHM 332), ADF & TEST

#### COMM 1 – ATIS (***.*) – SET ALTIMETER

#### COMM 2 – TWR (***.*)

#### NAV 1 and NAV 2 – SET AS REQUIRED (SRH 109.95, GWC 114.75).

#### Marker beacon lights – TEST.

#### Attitude indicator  – CHECK.

#### DI – SYNCHRONISE WITH COMPASS.

#### Fuel selectors – ON MAIN TANKS.

#### Taxiing light – ON.

#### Clearance - REQUEST TAXI FOR ……….

## Taxiing.

#### Brakes – CHECK OPERATION GENTLY.

#### Rudder and nose gear steering – FULL AND FREE MOVEMENT.

#### Flight instruments – TC INDICATES TURN RIGHT, BALL OUT TO THE LEFT, DI INCREASES, COMPASS ALSO. TC INDICATES TURN LEFT, BALL OUT TO THE RIGHT, DI DECREASES, COMPASS ALSO. AI STEADY, ALTIMETER STEADY.

## Power.

#### Position – INTO WIND AND NOSEWHEEL STRAIGHT.

#### Brakes – SET.

#### Left engine:

#### Throttle – SET 1500 RPM, CHECK BRAKES HOLDING.

#### Alternators – CHECK.

#### Vacuum system – CHECK (4.75 to 5.25 INCHES MERCURY)

#### Propeller – CHECK FOR FEATHERING TO 1000 RPM.

#### Magnetos – CHECK EACH MAGNETO DROP DOES NOT EXCEED 100 RPM. DIFFERENCE NOT TO EXCED 50 RPM.

#### Engine T’s & P’s – CHECK GREEN ARC.

#### Throttles – 900 RPM

## Altimeter settings.

#### Circuits – SET QFE.

#### Local – SET AERODROME QNH.

#### Cruise – SET ZONE/TMA/REGIONAL QNH BELOW 3000’. ABOVE TRANSITION LEVEL SET QNE.

#### Circuit join – SET QFE.

## Pre takeoff.

#### Trimmers – SET TAKEOFF.

#### Throttle – FRICTION NUTS FINGER TIGHT.

#### Mixtures – RICH.

#### Magnetos – ALL ON.

#### Propellers – FULL FORWARD

#### Fuel – RECHECK ON MAIN TANKS, AUXILIARY FUEL PUMPS ON.

#### Flaps - UP.

#### Gyros – AI ERECT, DI SYNCHRONISED, PITOT HEAT AS REQUIRED.

#### Instruments – NAVAIDS SET, TRANSPONDER STANDBY.

#### Annunciator panel – CLEAR.

#### Hatches – DOOR LOCKED, LIGHT OUT, WINDOW VENT CLOSED.

#### Harnesses – SECURE.

#### Controls – FULL AND FREE MOVEMENT.

#### Clearance – CALL READY FOR DEPARTURE.

## During take off.

#### Throttles – FULL OPEN (Approx. 39.5”/2275 RPM).

#### Directional control – MAINTAIN WITH RUDDER.

#### ASI – Increasing.

#### Engine T’S & P’S – NORMAL.

#### Elevator control – Raise nosewheel at 100 KIAS.

#### Minimum control speed – 82 KIAS (Vmc)

#### Break ground at 103 KIAS.

## After take off.

#### Brakes – APPLY MOMENTARILY.

#### Landing gear – UP (CHECK RED LIGHT OFF).

#### Throttles – REDUCE TO 32.5” Hg.

#### Propellers – REDUCE TO 1950 RPM.

#### Lookout – SELECT POINT LEFT OF NOSE.

#### Climb – 110 KIAS (Vy)

#### Auxiliary fuel pumps – CHECK ON

## Engine failure after take-off.

#### If engine failure occurs prior to take off, cut power and stop. If it occurs after take off but with enough runway ahead to land safely, cut power and land.

#### If a safe landing cannot be made, do the following:

#### Maintain directional control.

#### Apply full power to the good engine.

#### Landing gear – UP.

#### Inoperative Engine:

###### Throttle – CLOSE

###### Mixture – ICO

###### Propeller – FEATHER.

#### Establish Bank - 5º toward operative engine.

#### Climb to clear obstacle – 103 KIAS (Vxse).

#### Climb – 108 KIAS (Vyse).

#### Rudder – TRIM AS NECESSARY.

#### Aileron – TRIM (5º bank toward operative engine).

#### Inoperative engine – SECURE as follows:

###### Fuel – OFF

###### Auxiliary fuel pump – OFF

###### Magneto switches – OFF

###### Alternator  switch - OFF

#### Land as soon as practicable.

## Cruise.

#### Throttles –17 to 32.5 ” Hg.

#### Propellers - 1600 to1950 RPM (USE LOWEST ENGINE SPEED IN GREEN ARC THAT WILL GIVE SMOOTH ENGINE OPERATION WITHOUT EVIDENCE OF LABOURING).

#### Mixtures – 185.0 lbs./Hr 1950 RPM, 25” Hg, 53.8% @ 5000’, 166 KTAS.

#### Mixtures – 235.2 lbs./Hr 1950 RPM, 31” Hg, 69.8% @ 5000’, 186KTAS.

#### Mixtures – Best Economy, 55% power or less and lean to peak EGT or 6lbs/Hr per engine less than shown on performance charts.

#### Auxiliary Fuel Pumps:

###### Main Tanks – ON

###### When changing fuel tanks – LOW

###### Auxiliary tanks - LOW

#### Fuel Selectors – AS DESIRED (feel for detents)

###### After 90 minutes of flight  - AUXILIARY TANKS.

###### Usable fuel quantity is based on level flight.

###### If wing locker fuel tanks are installed, fuel selectors – MAIN TANK or, after wing locker tank contents are transferred and main tank quantity is less than 120 pounds each - AUXILIARY TANK.

###### If wing locker tanks are installed, crossfeed – SELECT as required to maintain fuel balance after wing locker tank fuel transfer.

#### Rest cabin altitude control to destination field pressure altitude plus 500 feet (inner scale)  or cruise altitude plus 500 feet (outer scale) whichever gives the highest cabin altitude (Optional System).

#### Cabin rate control – ARROW UP.

#### If Cabin Altitude Light Illuminates (cabin altitude above 10 000 feet) – Monitor cabin altimeter and use supplementary oxygen and/or DESCEND.

#### Trim tabs - ADJUST.

## Letdown

#### Power – AS REQUIRED

#### Mixture – ADJUST for smooth operation with gradual enrichment as altitude is lost.

#### Cabin altitude Control – SET SLOWLY.

#### Cabin rate Control – SET to reach selected cabin altitude at approximately the same time the aircraft reaches field pressure plus 500 feet (Optional system).

## Before landing.

#### Fuel Selectors – 	Left engine – LEFT MAIN,

#### Right engine – RIGHT MAIN

#### Auxiliary Fuel Pumps - ON

#### Alternate Air Controls – Check IN.

#### Mixtures – FULL RICH or lean as required for smooth operation.

#### Propellers – FULL FORWARD.

#### Flaps – 15º DOWN BELOW 175 KCAS.

#### Landing gear – DOWN BELOW 145 KCAS.

#### Landing gear - THREE GREENS, Unlocked light - OFF.

#### Cabin differential pressure - ZERO

#### Flaps - 15º to 45º DOWN BELOW 145 KCAS.

#### Minimum multi-engine approach speed – 103 KIAS.

#### Minimum single engine control speed – 82KIAS.

## Landing.

#### Touchdown – MAIN WHEELS FIRST.

#### Landing Roll – Lower nose wheel gently.

#### Brakes – AS REQUIRED.

## Go around (Multi-Engine).

#### Increase engine speed to 2275 RPM and apply full throttle if necessary

#### Airspeed – MINIMUM 103 KIAS, Note: Vmc is  82 KIAS.

#### Flaps – UP TO 15º.

#### Trim aircraft for climb.

#### Flaps – UP AS SOON AS ALL OBSTACLES ARE CLEARED.

## After landing.

#### Auxiliary Fuel Pumps – LOW (during landing roll).

#### Position – CLEAR OF ACTIVE RUNWAY AND STOP.

#### Flaps – UP.

## Securing Aircraft.

#### Auxiliary Fuel Pumps – OFF.

#### Avionics Bus Switch – OFF.

#### All switches except battery, Alternator and Magneto Switches – OFF.

#### Throttles – IDLE.

#### Mixtures – IDLE CUT-OFF.

#### Magneto switches – OFF, AFTER ENGINES STOP.

#### Battery and Alternators – OFF.

#### Parking brake – SET.

#### Control Lock(s) – INSTALL.

#### Fuel selectors – OFF.

#### Time – RECORD HOBBS METER READING.

#### Cabin door – CLOSE.

## Emergency gear extension.

#### Gear down lights – CHECK, IF NECESSARY DIM BY TURNING MASTER DIMMING SWITCH TO NIGHT AND UTILIZING COMPASS RHEOSTAT.

#### Landing gear motor circuit breaker – CHECK WITH LANDING GEAR SWITCH DOWN (ALLOW 3 MINUTES TO COOL BEFORE RESETTING).

#### If landing gear motor circuit breaker is NOT tripped – PULL.

#### Landing gear switch – NEUTRAL.

#### Pilot’s seat – TILT full aft or raise.

#### Handcrank – EXTEND AND LOCK.

#### Rotate crank – CLOCKWISE FOUR TURNS PAST THE POINT WHERE GEAR DOWN LIGHTS ARE ILLUMINATED (ABOUT 54 TURNS)

#### Gear down lights – ON; Unlocked light – OFF.

#### Gear warning Horn – CHECK with throttle retarded.

#### Handcrank – PUSH BUTTON AND STOW.

#### As soon as practicable – LAND.

## Engine feathering in flight.

#### Inoperative Engine - SECURE

##### Throttle – CLOSE

##### Mixture – IDLE CUT-OFF.

##### Propeller – FEATHER.

##### Fuel SELECTOR – OFF.

##### Auxiliary fuel pump – OFF.

##### Magneto switches – OFF.

##### Alternator switch - OFF

#### Operative engine:

###### POWER – DECREASE IF BELOW 82 KIAS

###### POWER – ADJUST IF ABOVE 82 KIAS AS REQUIRED.

#### Mixture – ADJUST for power.

#### Fuel – MAIN TANK.

#### Auxiliary fuel pump – ON.

#### Rudder – TRIM AS NECESSARY.

#### Aileron – TRIM (5º bank toward operative engine).

#### Electrical load – DECREASE to minimum required.

#### As soon as practicable – LAND.

## Engine restart in flight (After  Feathering).

#### Magneto switches – ON.

#### Fuel selector – MAIN TANK.

#### Throttle – FORWARD ABOUT ONE INCH.

#### Mixture – FULL RICH.

#### Propeller –FORWARD of détente.

#### Starter switch – PRESS.

#### Primer switch – ACTIVATE.

#### Starter and Primer switches – RELEASE when engine fires.

#### Auxiliary fuel pump – ON.

#### Alternator switch – ON.

#### Power – INCREASE after cylinder head temperatures reach 200ºF.

## Range (no fuel reserves) in still air.

#### 73.5%	249.0 lbs./hr	4.1 hours	779 miles @ 5 000’.

#### 69.8%	235.2 lbs./hr	4.34 hours	806 miles @ 5 000’.

#### 64.8%	218.5 lbs./hr	4.67 hours	840 miles @ 5 000’.

#### 59.6%	201.8 lbs./hr	5.05 hours	878 miles @ 5 000’.

#### 53.8%	185.0 lbs./hr	5.51 hours	914 miles @ 5 000’.

## Maximum Landing Weight – 7 200 lbs.

#### Top speed				245Kts @ 18 000’

#### 75% cruise speed			201Kts @ 10 000’

#### Take off run (short field)		820’.

#### Take off run - 50’ obstacle.		2 387’.

#### Accelerate–Stop distance.		3 700’ @ SL.

#### Best climb – Single engine.		109 KIAS	345 fpm @ SL.

#### Landing roll (Short field)		720’.

#### Landing roll – 50’ obstacle.		2178’.

#### Service ceiling.			31 100’.

#### Single engine absolute ceiling.	13 000’.

#### 7/29/2026 input.doc