# PA-23-250  AZTEC “C” CHECKLIST

## Preflight inspection.

#### Aircraft documents – VALID AND COMPLETE.

#### Control locks – REMOVE.

#### Parking brake – CHECK ON.

#### Switches – OFF (MAGNETOS, FUEL PUMPS, LIGHTS, NAV/COM RADIOS, TRANSPONDER, ADF AND AUTOPILOT).

#### Circuit breakers – IN.

#### Master switch – ON.

#### Beacon – ON.

#### Landing gear lights – THREE GREENS.

#### Fuel quantities– CHECK.

#### Master switch – OFF.

#### Fire extinguisher – SECURE.

#### First aid kit – IN PLACE.

#### Flaps – HAND PUMP DOWN.

#### Fuel – SELECT INBOARD TANKS.

#### Heater fuel – OFF.

## Starboard fuselage.

#### Rear fuselage skin – CHECK CONDITION.

#### Aft baggage door - SECURE.

#### ELT aerial - SECURE.

#### Stabilator – CHECK CONDITION, FREEDOM OF MOVEMENT AND TAB POSITION.

#### Rudder – CHECK FIXINGS AND HINGES SECURE.

#### Rotating beacon light and NAV. aerials – CHECK SECURITY.

#### Tie down rope – REMOVE.

## Port fuselage.

#### Fuselage skin - CONDITION.

#### Comm. aerial - SECURE.

#### ADF sensing and loop aerial - CHECK.

## Port wing.

#### Flap – CHECK SECURITY AND ATTACHMENT.

#### Upper wing surface – CHECK CONDITION AND FREE FROM ICE, SNOW AND FROST.

#### Aileron – CHECK CONDITION AND FREEDOM OF MOVEMENT.

#### Navigation and strobe lights – SECURE.

#### Leading edge – CHECK CONDITION.

#### Stall warning vane – CHECK.

#### Lower wing surface – CHECK CONDITION AND CLEANLINESS.

#### Pitot tube – CHECK CLEAR.

#### Tie down – REMOVE.

#### Fuel tanks – CHECK CONTENTS, SECURE CAPS.

#### Engine – OIL LEVEL BETWEEN 9 AND 12 QUARTS.

#### Propeller  – CHECK FOR NICKS, DAMAGE AND SECURITY.

#### Left engine cowling – CHECK SECURITY.

#### Tyre  - CHECK CONDITION, INFLATION AND CREEP.

#### Oleo piston – CHECK EXTENSION 3.0”.

#### Fuel tank sumps and lines – SAMPLE/DRAIN FUEL STRAINERS.

#### Cowl flaps, wheel well doors – CHECK CONDITION

## Nose.

#### Tyre  - CHECK CONDITION, INFLATION AND CREEP.

#### Oleo piston – CHECK EXTENSION 3.0”.

#### Torque link – SECURITY OF LINKAGES.

#### Windscreen – CHECK CONDITION AND CLEANLINESS.

#### Landing light – SECURE.

#### Forward baggage door – SECURE.

## Starboard wing.

#### Engine – OIL LEVEL BETWEEN 9 AND 12 QUARTS.

#### Propeller – CHECK FOR NICKS, DAMAGE AND SECURITY.

#### Right engine cowling – CHECK SECURITY.

#### Tie down rope – REMOVE.

#### Fuel tank sumps and lines – SAMPLE/DRAIN FUEL STRAINERS.

#### Tyre  - CHECK CONDITION, INFLATION AND CREEP.

#### Oleo piston – CHECK EXTENSION 3.0”.

#### Cowl flaps, wheel well doors – CHECK CONDITION.

#### Lower wing surface – CHECK CONDITION AND CLEANLINESS.

#### Fuel tanks – CHECK CONTENTS, SECURE CAPS.

#### Upper wing surface – CHECK CONDITION AND FREE FROM ICE, SNOW AND FROST.

#### Leading edge – CHECK CONDITION.

#### Wing tip navigation and strobe lights – SECURE.

#### Aileron – CHECK CONDITION, SECURITY AND FREEDOM OF MOVEMENT.

#### Flap – CHECK SECURITY AND ATTACHMENT.

#### Loose equipment – REMOVE OR SECURE.

## Before starting engines.

#### Seats – ADJUSTED, HARNESSES SECURE.

#### Flying controls – FULL AND FREE MOVEMENT IN THE CORRECT SENSE.

#### Cabin door – CLOSE AND LOCK.

#### Parking brake – SET.

#### Landing gear selector – DOWN.

#### Trimmer – SLIGHTLY AFT OF NEUTRAL.

#### Flap selector – NEUTRAL.

#### Cowl flaps – OPEN.

#### Cross Feed  – OFF (drain cross feed valve).

#### Fuel – SELECT INBOARD TANKS, CHECK CONTENTS.

#### Heater controls  – AS REQUIRED.

#### Alternate induction air – NORMAL.

#### Static pressure valve - NORMAL.

#### Voltage regulator selector – MAIN.

#### Hydraulic hand pump – STOWED.

#### Compass – SENSIBLE HEADING, NO DISCOLORATION OR BUBBLES.

#### VSI – Zero  200’ (Note error).

#### Instruments and gauges – CLOCK ACCURATE, ASI VERTICAL, ALTIMETER.

#### Turn coordinator – FUNCTIONING.

#### OAT – SENSIBLE READING.

#### Circuit breakers – ALL IN.

#### Hobbs meter reading – RECORD.

## Starting engines.

#### Throttles – FULL OPEN.

#### Propellers – FULL FORWARD (Fine pitch).

#### Mixtures – IDLE CUT OFF.

#### Master switch  – ON.

#### Magneto switches – ALL ON.

#### Fuel pump (left) – ON, CHECK PRESSURE READING.

#### Mixture (left) – FULL RICH UNTIL FUEL FLOW INDICATED, THEN IDLE CUT OFF (Engine is now primed).

#### Throttle (left) – SET 1/2” OPEN.

#### Area – LOOKOUT, SHOUT ‘CLEAR LEFT’.

#### Starter – OPERATE.

#### Mixture – FULL RICH WHEN ENGINE FIRES, RELEASE STARTER SWITCH.

#### Starter warning light – OUT.

#### Power – SET 1200 RPM.

#### Oil pressure – RISING WITHIN 30 SECONDS.

#### Alternator (left) – CHECK AMMETER READING, MAX. 30 AMPS.

#### Suction (left) – INDICATING.

#### Fuel pump (left) – OFF.

#### Magnetos (left) – CHECK FOR DEAD CUT.

## Starting right engine.

#### Repeat above items 6 to 17. Note that landing gear selector returns to neutral and that flaps raise and flap selector returns to neutral.

## Starting hot engine.

#### Master switch – ON.

#### Magneto switches – ON.

#### Fuel pump – OFF.

#### Throttle – SET 1/2” OPEN.

#### Mixture – IDLE CUT OFF.

#### Area – LOOKOUT, SHOUT ‘CLEAR PROP’.

#### Starter – OPERATE.

#### Mixture – FULL RICH WHEN ENGINE FIRES, RELEASE STARTER.

#### Starter warning light – OUT.

## Starting flooded engine.

#### Master switch – ON.

#### Magneto switches – ON.

#### Fuel pump – OFF.

#### Throttle – SET FULL OPEN.

#### Mixture – IDLE CUT OFF.

#### Area – LOOKOUT, SHOUT ‘CLEAR PROP’.

#### Starter – OPERATE.

#### Mixture – IDLE WHEN ENGINE FIRES THEN FULL RICH, RELEASE STARTER.

#### Starter warning light – OUT.

## Before taxiing.

#### Beacon – CHECK ON.

#### Strobe lights – ON.

#### Radios –ON (ADF to REC, COMM 1 and COMM 2 ON, NAV 1 and NAV 2 to IDENT, Transponder to STANDBY and AUDIO SWITCHES TO PHONES.

#### ADF – REC to tune and identify (SHM 332), ADF & TEST

#### COMM 1 – ATIS (125.3) – SET ALTIMETER

#### COMM 2 – TWR (123.15)

#### NAV 1 and NAV 2 – SET AS REQUIRED (SRH 109.95, GWC 114.75).

#### Marker beacon lights – TEST.

#### Attitude indicator  – CHECK.

#### DI – SYNCHRONISE WITH COMPASS.

#### Fuel selectors – ON EMPTIER TANKS.

#### Landing light – ON.

#### Clearance - REQUEST TAXI FOR ……….

## Taxiing.

#### Brakes – CHECK OPERATION GENTLY.

#### Rudder and nose gear steering – FULL AND FREE MOVEMENT.

#### Flight instruments – TC INDICATES TURN RIGHT, BALL OUT TO THE LEFT, DI INCREASES, COMPASS ALSO. TC INDICATES TURN LEFT, BALL OUT TO THE RIGHT, DI DECREASES, COMPASS ALSO. AI STEADY, ALTIMETER STEADY.

## Power.

#### Position – INTO WIND AND NOSEWHEEL STRAIGHT.

#### Parking brake – SET.

#### Left engine:

#### Fuel selector – CHANGE TO FULLER TANK.

#### Mixture – RICH.

#### Throttle – SET 2200 RPM, CHECK BRAKES HOLDING.

#### Propeller – EXERCISE TWICE.

#### Magnetos – CHECK EACH MAGNETO DROP DOES NOT EXCEED 175 RPM. DIFFERENCE NOT TO EXCEDE 50 RPM.

#### Engine T’s & P’s – NORMAL.

#### Suction – 4.8” to 5.1”

#### Alternator – AMMETER READING NORMAL (MAX. 30 AMPS.)

#### Fuel flow - NORMAL

#### Throttle – 1500 RPM

#### Propeller feathering – CHECK, DO NOT DROP BELOW 1 000 RPM.

#### Throttle – CLOSE, CHECK IDLING BETWEEN 500 AND 700 RPM, RESET 1200 RPM. DO NOT IDLE BELOW 1200 RPM.

#### Right engine – Same as left engine

## Altimeter settings.

#### Circuits – SET QFE.

#### Local – SET AERODROME QNH.

#### Cruise – SET ZONE/TMA/REGIONAL QNH BELOW 3000’. ABOVE TRANSITION LEVEL SET QNE.

#### Circuit join – SET QFE.

## Pre takeoff.

#### Trimmer – SET TAKEOFF.

#### Throttle – FRICTION NUTS FINGER TIGHT.

#### Mixtures – RICH.

#### Magnetos – ALL ON.

#### Propellers – FULL FORWARD

#### Fuel – ON FULLEST TANKS, FUEL PUMPS ON.

#### Flaps - UP.

#### Cowl flaps – OPEN.

#### Gyros – AI ERECT, DI SYNCHRONISED, PITOT HEAT AS REQUIRED.

#### Instruments – NAVAIDS SET, TRANSPONDER STANDBY.

#### Hatches – DOOR LOCKED, LIGHT OUT, WINDOW VENT CLOSED.

#### Harnesses – SECURE.

#### Controls – FULL AND FREE MOVEMENT.

#### Clearance – CALL READY FOR DEPARTURE.

## During take off.

#### Throttles – FULL OPEN (Approx. 29”/2575 RPM).

#### Directional control – MAINTAIN WITH RUDDER.

#### ASI – Increasing.

#### Engine T’S & P’S – NORMAL.

#### Rotate – 80 MPH (Vmca)

## After take off.

#### Brakes – APPLY MOMENTARILY.

#### Landing gear – UP when safe landing cannot be accomplished on remaining runway.

#### Throttles – REDUCE TO 24” Hg.

#### Propellers – REDUCE TO 2400 RPM.

#### Lookout – SELECT POINT LEFT OF NOSE.

#### Climb – 120 MPH (Vy) – 135 MPH FOR CRUISE CLIMB.

#### Fuel pump – OFF ABOVE CIRCUIT HEIGHT, CHECK FUEL FLOW NORMAL.

## Engine failure after take-off.

#### If engine failure occurs prior to take off, cut power and stop. If it occurs after take off but with enough runway ahead to land safely, cut power and land.

#### If a safe landing cannot be made, do the following:

#### Maintain directional control.

#### Apply full power to the good engine.

#### Landing gear – UP.

#### Flaps – UP.

#### Feather dead engine.

#### Climb – 102 MPH (Vyse). Note: If the left engine fails, the landing gear and flaps must be pumped up by hand.

#### Rudder – TRIM AS NECESSARY.

#### Land as soon as practicable.

## Cruise.

#### Throttles – 20” Hg. (Approx. 55% power).

#### Propellers – 2200 RPM.

#### Cowl flaps – CLOSED.

#### Mixtures – SET AS REQUIRED.

## Top of climb.

#### Fuel – CHECK CONTENTS, SELECT APPROPRIATE TANK.

#### Engine – T’s & P’S IN THE GREEN.

#### Mixture – LEAN AS REQUIRED.

#### DI – SYNCHRONISE WITH COMPASS.

#### Orientation – SELECT REFERENCE POINT.

## En-route/Airfield approach.

#### Fuel – CHECK CONTENTS, SELECT APPROPRIATE TANK.

#### Radios – TUNED AND CORRECT.

#### Engine – T’s & P’s NORMAL.

#### DI – SYNCHRONISE WITH COMPASS.

#### Altimeter– SET AS REQUIRED.

## Pre-stalling.

#### Height – SUFFICIENT TO RECOVER BY 2000’ agl DUAL.

#### Airframe – FLAPS AS REQUIRED.

#### Security – NO LOOSE ARTICLES, HATCH CLOSED AND LOCKED, HARNESSES SECURE.

#### Engine – T’s & P’S NORMAL, MIXTURES RICH, FULLER TANKS SELECTED, FUEL PUMPS ON.

#### Location – CLEAR OF ACTIVE AIRFIELDS, BUILT-UP AREAS, CONTROLLED AIRSPACE.

#### Lookout– CLEAR OF OTHER AIRCRAFT AND CLOUD BOTH HORIZONTALLY AND VERTICALLY – CLEARING TURNS LEFT AND RIGHT THROUGH 90.

## After stalling.

#### Fuel – CHECK CONTENTS, SELECT APPROPRIATE TANKS, FUEL PUMPS OFF.

#### Radios – TUNED AND CORRECT.

#### Engine – T’s & P’S NORMAL.

#### DI – SYNCHRONISE WITH COMPASS.

#### Altimeter– SET QNH.

## Pre-landing checks.

#### Brakes – OFF.

#### Undercarriage – DOWN AND LOCKED.

#### Mixtures – FULL RICH.

#### Propellers – 2400 RPM.

#### Throttles – 20” Hg.

#### Fuel – ON FULLEST TANKS, FUEL PUMPS ON.

#### Instruments - T’s & P’S NORMAL, SUCTION INDICATING, AMMETER READING NORMAL, DI SYNCHRONISED WITH COMPASS.

#### Cowl flaps – AS REQUIRED (OPEN FOR TOUCH AND GO LANDINGS).

#### Hatches – CLOSED AND LOCKED.

#### Harnesses – SECURE.

#### Flaps – ¼ DOWN (160 MPH MAX).

#### Landing gear – DOWN (150 MPH MAX).

#### Propellers – FULL FORWARD WHEN LANDING ASSURED.

#### Landing gear - THREE GREENS.

#### Flaps – FULLY DOWN WHEN LANDING ASSURED (Vfe 125 MPH).

## Go around.

#### Throttles – FULL POWER.

#### Airspeed – MINIMUM 80 MPH (Vmc).

#### Flaps – UP TO HALF.

#### Landing gear – UP WHEN CLIMB ESTABLISHED.

#### Flaps – UP.

#### Power – 24”/2400 RPM.

## After landing.

#### Position – CLEAR OF ACTIVE RUNWAY AND STOP.

#### Flaps – UP.

#### Heater – OFF TWO MINUTES BEFORE SHUT DOWN.

#### Fuel pumps – OFF.

#### Cowl flaps – OPEN.

#### Throttle friction nut – LOOSE.

#### Trim – NEUTRAL.

#### Landing lights – OFF.

#### Electrics - NON ESSENTIAL LOADS OFF.

#### Clearance – CALL GROUND CONTROL/TOWER.

## Shut down.

#### Position – INTO WIND, NOSE WHEEL STRAIGHT.

#### Parking brake – SET.

#### Throttles – SET 1200 RPM.

#### Radio – OFF.

#### Magnetos – CHECK DEAD CUT AND LIVE MAG.

#### Mixtures – IDLE CUT OFF.

#### Throttles – FULLY AFT.

#### Magnetos – OFF WHEN ENGINES STOP, KEY OUT.

#### Master – OFF.

#### Fuel selectors – OFF.

#### Time – RECORD HOBBS METER READING.

#### Seats – FULLY BACK.

#### Cockpit – CLEAN AND HARNESSES TIDY FOR NEXT PILOT.

#### Aircraft – SECURE.

## Emergency gear extension.

#### Gear down lights – CHECK, IF NECESSARY TURN PANEL LIGHTS OFF.

#### If left engine inoperative – LANDING GEAR HANDLE DOWN, USE HAND PUMP TO LOCK (30 TO 40 STROKES NEEDED TO LOWER LANDING GEAR, 12 TO LOWER FLAPS)

#### If hydraulic system inoperative – USE EMERGENCY GEAR EXTENDER. LANDING GEAR HANDLE DOWN, SHARPLY PULL RING UNDER DOOR BENEATH PILOT’S SEAT.

## Emergency feathering.

#### Operating engine –  FULL POWER OR AS REQUIRED.

#### Dead engine throttle – CLOSED

#### Dead engine propeller – FULL AFT.

#### Dead engine mixture – IDLE CUT OFF.

#### Dead engine magneto switches – OFF.

#### Dead engine fuel pump – OFF.

#### Dead engine fuel SELECTOR – OFF.

#### Rudder trim – AS REQUIRED.

#### Cowl flaps – CLOSED ON DEAD ENGINE, AS REQUIRED ON OPERATING ENGINE

#### Cross feed – AS REQUIRED.

## Unfeathering procedure.

#### Fuel – ON.

#### Magneto switches – ON.

#### Prime if necessary, Fuel pump – OFF.

#### Throttle – OPEN ½ “.

#### Propeller – FULL FORWARD.

#### Mixture – FULL RICH.

#### Starter – OPERATE UNTIL PROP. ROTATES BY ITSELF (130 MPH AIDS WINDMILLING).

#### Throttle – IDLE UNTIL T’s & P’s BEGIN TO RISE.

#### Engines – RESYNCHRONIZE.

## Fuel system – Takeoff and landing.

#### Fuel selectors – ON FULLEST TANKS.

#### Cross feed valve - OFF.

#### Fuel pumps – ON.

## Fuel system – Cruise.

#### Fuel selectors – ON FULLEST TANKS.

#### Cross feed valve - OFF.

#### Fuel pumps – OFF.

## Fuel system – Emergency operation.

#### Cross feed - ON

#### Fuel selector of inoperative engine – ON FULLEST TANK.

#### Fuel pump of inoperative engine – ON.

#### Fuel selector of operating engine – OFF.

#### When fuel from tanks of inoperative engine is exhausted return to operating engine tanks as follows:

#### Fuel pump of operating engine – ON.

#### Fuel selector of operating engine – ON FULLEST TANK.

#### Cross feed valve - OFF.

#### Fuel pump of inoperative engine – OFF.

## Range without reserves in still air.

#### 75%	27.4 USGPH	5.1 hours	1 055 miles @ 7 500’.

#### 65%	23.8 USGPH	5.9 hours	1 170 miles @ 10 000’.

#### 55%	21.0 USGPH	6.7 hours	1 220 miles @ 10 000’.

#### 45%	17.8 USGPH	7.9 hours	1 285 miles @ 10 000’.

## Maximum Landing Weight – 4 940 lbs.

#### Top speed		216 MPH

#### 75% cruise speed	206 MPH @ 7 500’

#### 75% cruise speed			193 MPH @ SL.

#### 65% cruise speed			201 MPH @ 10 000’.

#### Take off run (short field)		820’.

#### Take off run - 50’ obstacle.	1250’.

#### Accelerate–Stop distance.		2200’ @ SL.

#### Vxse (Left engine inoperative)	102 MPH	240 fpm @ SL.

#### Vyse				138 MPH

#### Landing roll (Short field)		860’.

#### Landing roll – 50’ obstacle.	1680’.

#### Absolute ceiling.			21 100’.

#### Service ceiling.			19 800’.

#### Single engine absolute ceiling.	6 400’ (Left engine out).

#### Single engine service ceiling.	5 000’ (Left engine out).

#### 8-Nov-97 PA-23-250 CHECKLIST.doc